The DVB-RCS MIB
draft-combes-ipdvb-mib-rcs-00.doc

Status of this Memo

By submitting this Internet-Draft, each author represents that       any applicable patent or other IPR claims of which he or she is       aware have been or will be disclosed, and any of which he or she       becomes aware will be disclosed, in accordance with Section 6 of       BCP 79.

This document may not be modified, and derivative works of it may not be created, except to publish it as an RFC and to translate it into languages other than English, or to extract section 3 as-is for separate use.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at
     http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at
     http://www.ietf.org/shadow.html

This Internet-Draft will expire on September 31, 2007.

Abstract

This document describes MIB module for DVB-RCS. It defines a set of new MIB entities to characterise the behaviour and performance of network layer entities deploying DVB-RCS.

Table of Contents

1. Introduction	2

2. Conventions used in this document	2

2.1. MIB Module	3

3. Definitions	4

4. Security Considerations	86

5. IANA Considerations	86

6. Conclusions	86

7. Acknowledgments	86

8. References	87

8.1. Normative References	87

8.2. Informative References	87

9. Author's Addresses	88

Intellectual Property Statement	88

10. Disclaimer of Validity	88

11. Copyright Statement	89

# Introduction

The SatLabs Group is an international non-profit EEIG (European Economic Interest Grouping) committed to large-scale adoption and deployment of the DVB-RCS standard [ETSI-RCS]. SatLabs members are service providers, satellite operators, system integrators, terminal manufacturers and technology providers with an interest in DVB-RCS.

Since its creation in 2001, the main goal of the SatLabs Group has been to achieve interoperability between DVB-RCS terminals and systems. Therefore, the Group has defined the SatLabs Qualification Program which provides an independent certification process for DVB-RCS Terminals based on System Recommendations defined by SatLabs.

To enhance products interoperability, beyond the physical and MAC layers mechanisms defined in the DVB-RCS standard, SatLabs has expanded its Recommendations in the field of DVB-RCS terminal management [SATLABS].

As part of this effort, SatLabs has drafted a common SNMP Management Information Base (MIB) for DVB-RCS terminals. This MIB is defined in the current document.

# Conventions used in this document

This memo defines a portion of the Management Information Base (MIB) for use with network management protocols in the Internet community.

For a detailed overview of the documents that describe the current Internet-Standard Management Framework, please refer to section 7 of RFC 3410 [RFC3410].

Managed objects are accessed via a virtual information store, termed the Management Information Base or MIB. MIB objects are generally accessed through the Simple Network Management Protocol (SNMP). Objects in the MIB are defined using the mechanisms defined in the Structure of Management Information (SMI).  This memo specifies a MIB module that is compliant to the SMIv2, which is described in STD 58, RFC 2578 [RFC2578], STD 58, RFC 2579 [RFC2579] and STD 58, RFC 2580 [RFC2580].

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT" "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in RFC 2119. [RFC2119].

## MIB Module

The MIB module in this document uses the following IANA-assigned OBJECT IDENTIFIER values:

+------------+-------------------------+

| Descriptor | OBJECT IDENTIFIER value |

+------------+-------------------------+

|dvbrcsIfMib |{ mib-2 transmission 239 }      |

+------------+-------------------------+

Table 1: Object Identifiers for the MIB

These values have been assigned for this MIB under the 'mib-2.transmission' subtree.

# Definitions

dvbrcsIfMib DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, transmission, NOTIFICATION-TYPE

FROM SNMPv2-SMI

TEXTUAL-CONVENTION, DisplayString,

PhysAddress, RowStatus    FROM SNMPv2-TC;

dvbrcsIfMib MODULE-IDENTITY

LAST-UPDATED "200701111200Z"

ORGANIZATION "The SatLabs Group"

CONTACT-INFO

"The SatLabs Group

Web:	www.satlabs.org

E-mail: info@satlabs.org      "

DESCRIPTION

"SatLabs MIB subtree. Draft version A"

REVISION "200701111200Z"

DESCRIPTION

"Draft revision A of this MIB module.

Number in the transmission tree 239"

::= { transmission 239 }

rcstSystem    	OBJECT IDENTIFIER ::= {dvbrcsIfMib 1}

rcstConfig    	OBJECT IDENTIFIER ::= {dvbrcsIfMib 2}

rcstStatus    	OBJECT IDENTIFIER ::= {dvbrcsIfMib 3}

rcstAction    	OBJECT IDENTIFIER ::= {dvbrcsIfMib 4}

installation    OBJECT IDENTIFIER ::= {rcstSystem 1}

options		    OBJECT IDENTIFIER ::= {rcstSystem 2}

network         OBJECT IDENTIFIER ::= {rcstConfig 1}

rtnLink         OBJECT IDENTIFIER ::= {rcstConfig 2}

fwdLink         OBJECT IDENTIFIER ::= {rcstConfig 3}

qos          	OBJECT IDENTIFIER ::= {rcstConfig 4}

--=============================================================================

---    Definition of  rcstSystem.installation group

--=============================================================================

mibVersion		OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"This object is the MIB version. The first version is 1.0."

::= {installation 1}

location		OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Physical location of the ODU antenna expressed as Longitude,

Latitude and Altitude. The format used shall be NMEA 0183,

as shown below:

Latitude:   llll.ll,a    (a=N or S)

Longitude:  yyy.yy,b     (b=E or W)

Altitude:  xxxx.x        (in meters)

Examples:

4916.46,N    Latitude 49 deg. 16.45 min. North

12311.12,W   Longitude 123 deg. 11.12 min. West

545.4,M      Altitude, Metres, above mean sea level

This location, and the satellite position are used to

calculate the RCST-satellite path delay.

Note: The system.sysLocation object of MIB-II provides

physical location of the IDU unit."

::= {installation 2}

oduAntennaSize	  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object gives the diameter of the antenna. This value

shall be given in centimeter. Defined at Installation. The

object can be used in conjunction with environmental

requirements. "

::= {installation 3}

oduAntennaGain	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the antenna peak gain of the ODU and

shall be defined by the installer. The gain shall be given

in tenth of dBi for more flexibility, i.e. 46.5 dBi will be

represented by 465. Defined at installation."

::= {installation 4}

oduSspa	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the SSPA installed in the ODU and

shall be defined by the installer. The power shall be given

in tenth of Watt for more flexibility, i.e. 0,5 W will be

represented by 5, 1 W by 10 and 2 W by 20.

Defined at installation."

::= {installation 5}

oduTxType	OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of transmitter installed in the ODU."

::= {installation 6}

oduRxType	OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of LNB installed in the ODU, with

information such as vendor type, ouput type (single, twin,

quad,Ö), etc.. "

::= {installation 7}

oduRxBand	OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes whether High Band or Low Band is selected in

the LNB. Specifying High Band results in activation of a

18-26 kHz tone with 0.4-0.8 Vpp in the Rx IFL cable:

(0)	- High Band

(1)	- Low Band "

::= {installation 8}

oduRxLO 	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Frequency of LNB Local Oscillator (in 100 Hz)"

::= {installation 9}

oduTxLO		OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency of BUC Local Oscillator (in 100 Hz) "

::= {installation 10}

antennaAlignmentState  OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Indicates the state:

(1)-Start;

(2)-Deny;

(3)-Continue;

(4)-Stop;

(5)-Success;

(6)-Fail"

::= {installation 11}

cwFrequency OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency at which the carrier is put up (in 100 Hz).

Minimum required precision is 1 kHz."

::= {installation 12}

cwMaxDuration OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time after which the carrier must be put down (in seconds)"

::= {installation 13}

cwPower	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specified in tenth of dBm. This parameter allows for fine

(manual) tuning of the power setup. Minimum required precision

is 1 dBm. It must be configurable while CW signal is on. Once

the power has been properly adjusted, the power setup is stored

in rcstConfigRtnLinkDefIfLevel."

::= {installation 14}

co-PolReading OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Pol measured value (in 0.1 dB)"

::= {installation 15}

x-PolReading	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Pol measured value (in 0.1 dB)"

::= {installation 16}

co-PolTarget OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Pol target (in 0.1 dB) "

::= {installation 17}

x-PolTarget		OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Pol target (in 0.1 dB)"

::= {installation 18}

standByDuration	OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time to wait in stand-by mode (in seconds)"

::= {installation 19}

targetEsNO		OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This value describes the wanted Es/N0 value that enables

operation of the return link with the required error

performance. The values shall be given in tenth of dB and

the initial value shall be equal to 7 dB. The range shall be

from 0 dBm to 31.5 dBm with a precision of 0,1 dB."

::= {installation 20}

--- rcstSystem.options group

mpegTrf         OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

" Indicates whether the MPEG_TRF SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 1}

coarseSync      OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

" Indicates whether the COARSE_SYNC SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 2}

wideHopp        OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

" Indicates whether the WIDE_HOPP SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 3}

fastHopp        OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the FAST_HOPP SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 4}

dynamicMfTdma   OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the Dynamic_MF_TDMA SatLabs option is

supported by the RCST:

Not supported (0)

Supported (1)"

::= {options 5}

routeId         OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the ROUTE_ID SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 6}

contentionSync  OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the CONTENTION_SYNC SatLabs option is

supported by the RCST:

Not supported (0)

Supported (1)"

::= {options 7}

qpskLow         OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the QPSKLOW SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 8}

m16Apsk          OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the 16APSK SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 9}

m32Apsk          OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the 32APSK SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 10}

normalFec       OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the NORMALFEC SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 11}

multiTs         OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the MULTITS SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 12}

gsTs            OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the GSTS SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 13}

enhQos          OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the ENHQOS SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 14}

pep             OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the PEP SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 15}

http            OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the HTTP SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 16}

ftp             OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the FTP SatLabs option is supported by

the RCST:

Not supported (0)

Supported (1)"

::= {options 17}

dhcp            OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

" Indicates whether the DHCP SatLabs option is supported by

the RCST:

Not supported (0)

Supported (1)"

::= {options 18}

dns  			OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the DNS SatLabs option is supported by

the RCST:

Not supported (0)

Supported (1)"

::= {options 19}

avbdcRep  			OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the AVBDC_REP SatLabs option is supported by

the RCST:

Not supported (0)

Supported (1)"

::= {options 20}

chIdStrict  OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the CHID_STRICT SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 21}

rcstPara        OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the RCST_PARA SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 22}

nlid            OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the NLID SatLabs option is supported by

the RCST:

Not supported (0)

Supported (1)"

::= {options 23}

snmpAll         OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the SNMPALL SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 24}

installLog      OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the INSTALL_LOG SatLabs option is supported

by the RCST:

Not supported (0)

Supported (1)"

::= {options 25}

fwdLinkStatus   OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the FWDLINKSTATUS SatLabs option is

supported by the RCST:

Not supported (0)

Supported (1)"

::= {options 26}

enhClassifier   OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether the ENHCLASSIFIER SatLabs option is

supported by the RCST:

Not supported (0)

Supported (1)"

::= {options 27}

---- rcstConfig.network group

oamIpAddr          OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"OAM IP Address of the RCST. This object used with both ip and

interfaces MIB-II subgroups determines uniquely the interface

through which OAM traffic is passing through.

Note that the OAM IP address may be statically or dynamically

assigned. It is system dependent whether the OAM IP address

and the Traffic IP address are the same address."

::= {network 1}

oamIpNetworkMask      OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Network Mask for the OAM IP Address."

::= {network 2}

oamIpAddrAssign     OBJECT-TYPE

SYNTAX             INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Identifies whether the OAM IP address is statically

('static' - 1) or dynamically ('dynamic' - 2) assigned."

::= {network 3}

lanIpAddress        OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the LAN interface"

::= {network 4}

lanIpMask           OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Mask for the LAN interface"

::= {network 5}

airInterfaceDefaultGateway   OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Default Gateway for the air interface"

::= {network 6}

dnsServerIpAddress    OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the DNS server in the NCC."

::= {network 7}

nccMgtIpAddress       OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the management server in the NCC."

::= {network 8}

configFileDownloadUrl  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST."

::= {network 9}

installLogFileDownloadUrl    OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file to download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST. The installation

log file can be created on the installer's computer

and downloaded to the RCST."

::= {network 10}

configFileUploadUrl   OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file upload.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname.Hostname can

only be used if DNS is supported by the RCST."

::= {network 11}

logFileUploadUrl        OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the event log file. It includes the protocol

type (tftp or ftp) and the associated server IP address

or hostname. Hostname can only be used if DNS is supported

by the RCST."

::= {network 12}

installLogFileUploadUrl  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file. It includes the

protocol type (tftp or ftp) and the associated server

IP address or hostname. Hostname can only be used if DNS is

supported by the RCST. The installation log file can be

retrieved from the RCST by the NCC or by the installer

via the LAN."

::= {network 13}

---- rcstConfig.rtnLink group

maxEirp            OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Maximum allowed EIRP in tenth of dBm on the return link."

::= {rtnLink 1}

defIfLevel        OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Default transmitted IF power level, specified in tenth of dBm,

out of the IDU for sending a CSC burst at RCST reboot or

power on. Applies to all signals transmitted by RCST

(CW and traffic) immediately."

::= {rtnLink 2}

---- rcstConfig.fwdLink group

----------StartConfig subgroup (1)

startConfig		OBJECT IDENTIFIER

::={fwdLink 1}

startConfigTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF StartConfigEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Lists Forward Links attachment points (e.g. different for installation and operation). The table describes the forward link parameters used for the start-up stream with the NCC."

::={startConfig 1}

startConfigEntry        OBJECT-TYPE

SYNTAX            StartConfigEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link StartConfig table."

INDEX   { popId }

::= { startConfigTable 1 }

StartConfigEntry ::= SEQUENCE {

popId 		Integer32,

freq        Integer32,

polar       INTEGER,

standard    INTEGER,

modulation  INTEGER,

rolloff     INTEGER,

symbRate    Integer32,

innerFec    INTEGER,

startConfigStatus   RowStatus

}

popId        OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Population identifier associated with the start-up

forward link:

-1: any (auto)

0-n: specific StartPopId"

::={startConfigEntry 1}

freq        OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Frequency of the start transponder carrying a

Network Information Table to which any RCST shall

trigger to acquire forward link. ts value shall be given

in multiple of 100 kHz."

::={startConfigEntry 2}

polar  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the start transponder

carrying an Network Information Table to which any RCST shall

trigger to acquire forward link:

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={startConfigEntry 3}

standard    OBJECT-TYPE

SYNTAX              INTEGER {

auto 	(-1),

dvbs 	(0),

dvbs2 	(1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the transmission standard applied on the start

transponder. The start transponder carries a

Network Information Table that the RCST uses for acquiring

the forward link signaling. Supported values are:

-1: any (auto)

0: DVB-S

1: DVB-S2

2-n: reserved "

::={startConfigEntry 4}

modulation    OBJECT-TYPE

SYNTAX              INTEGER  {

auto 	(0),

qpsk 	(1),

m8psk (2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the modulation used on the start transponder

carrying a NIT to which any RCST shall trigger to acquire

forward link.

0: any (auto)

1: QPSK

2: 8PSK

Other values shall be as specified in the NIT."

::={startConfigEntry 5}

rolloff    OBJECT-TYPE

SYNTAX              INTEGER {

auto 	(0),

r035 	(1),

r025 	(2),

r020 (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the roll-off applied on the start transponder.

The start transponder carries a Network Information Table

that the RCST uses for acquiring the forward link signaling. This object is only relevant when the transmission standard is 'DVB-S2'. Supported values are:

0: any (auto)

1: 0.35

2: 0.25

3: 0.20

4-n: reserved"

::={startConfigEntry 6}

symbRate    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the symbol rate on the start transponder

carrying a Network Information Table to which any RCST

shall trigger to acquire forward link. Its value shall be

given in multiple of 100 symbol/s."

::={startConfigEntry 7}

innerFec    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction used on the

start transponder carrying a NIT to which any RCST shall

trigger to acquire forward link.

Supported values are:

0: any (auto)

1: 1/2

2: 2/3

3: 3/4

4: 5/6

5: 7/8

6: 8/9

7: 3/5

8: 4/5

9: 9/10

10-14: reserved

15: No inner code "

::={startConfigEntry 8}

startConfigStatus    OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL  { active }

::={startConfigEntry 9}

--Status subgroup (2)

status		OBJECT IDENTIFIER

::={fwdLink 2}

statusTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF StatusEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the current status of Forward Link interfaces."

::={status 1}

statusEntry        OBJECT-TYPE

SYNTAX            StatusEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link Status table. Each entry is associated with a physical interface. A RCST SHALL support at least one entry."

INDEX   { ifIndex }

::= { statusTable 1 }

StatusEntry ::= SEQUENCE {

ifIndex					Integer32,

netId       			Integer32,

transmissionStandard  	INTEGER,

frequency    			Integer32,

polar  					INTEGER,

fecInner     			INTEGER,

symbolRate    			Integer32,

rolloff    				INTEGER,

modulation  			INTEGER,

fecFrame     			INTEGER,

pilot        			INTEGER,

ber          			Integer32,

cnr          			Integer32,

statusStatus   			INTEGER

}

ifIndex    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Cross reference to the interface table"

::={statusEntry 1}

netId      OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Interactive network identifier of the forward

link (from RMT)"

::={statusEntry 2}

transmissionStandard  OBJECT-TYPE

SYNTAX              INTEGER  {

dvbs 	(0),

dvbs2ccm 	(1),

dvbs2acm 	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specifies the transmission standard applied on the forward link. Supported values are (from RMT):

0: DVB-S

1: DVB-S2 using CCM

2: DVB-S2 using ACM

3: reserved"

::={statusEntry 3}

frequency    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency of the forward link (from RMT). Its value shall

be given in multiple of 100 kHz."

::={statusEntry 4}

polar    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the forward link

(from RMT):

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={statusEntry 5}

fecInner    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction of the

forward link (from RMT). This object is not used for DVB-S2.

Supported values are:

0: 1/2

1: 2/3

2: 3/4

3: 5/6

4: 7/8

5-14: reserved

15: No inner code"

::={statusEntry 6}

symbolRate    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specifies the symbol rate of the forward link (from RMT).

Its value shall be given in multiple of 100 symbol/s."

::={statusEntry 7}

rolloff    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specifies the roll-off applied on the forward link

(from RMT). This object is only relevant when the

transmission standard is 'DVB-S2'. Supported values are:

0: undefined

1: 0.20

2: 0.25

3: 0.35

4-1: reserved"

::={statusEntry 8}

modulation    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Indicates the modulation on the forward link.

0: undefined

1: QPSK (DVB-S)

2: 8PSK

3: reserved

For DVB-S2 with CCM, the MODCOD is fixed. The MODCOD value is

obtained from the PL Header of the PL frame every time the RCST

acquires the FL. The MODCOD corresponds to the values as defined

in the DVB-S2 standard:

1: QPSK 1/4

2: QPSK 1/3

3: QPSK 2/5

etc.

For DVB-S2 with ACM, the RCST updates the modulation object

from the PL Header every 10 sec."

::={statusEntry 9}

fecFrame    OBJECT-TYPE

SYNTAX              INTEGER {

shortframe	(0),

longframe	(1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the frame size of the forward link (from PL

header of DVB-S2). This object is only relevant when the

transmission standard is 'DVB-S2'. Supported values are:

0: Normal frame

1: Short frame

For DVB-S2 with CCM, the frame length is fixed. It is updated

from the PL Header every time the RCST acquire the FL.

For DVB-S2 with ACM, the RCST updates the frame length object

from the PL Header every 10 sec."

::={statusEntry 10}

pilot		OBJECT-TYPE

SYNTAX              INTEGER {

notused	(0),

used		(1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates if pilot mode is used on the forward link (from PL

header of DVB-S2). This object is only relevant when the

transmission standard is 'DVB-S2'. Supported values are:

0: Not used

1: In use

For DVB-S2 with CCM, the pilot is fixed. It is updated from

the PL Header every time the RCST acquire the FL.

For DVB-S2 with ACM, the RCST updates the pilot object from

the PL Header every 10 sec."

::={statusEntry 11}

ber		OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the RCST BER on the Forward Link in log10 units."

::={statusEntry 12}

cnr		OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the RCST CNR on the Forward Link in 0.1 dB units."

::={statusEntry 13}

statusStatus    OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={statusEntry 14}

---- rcstConfig.qos group

pktClassTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF PktClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the packet classification used in the DVB-RCS terminal. The number of entries is specified by PktClassIndex. "

::={qos 1}

pktClassEntry        OBJECT-TYPE

SYNTAX            PktClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the packet classification table. "

INDEX   { pktClassIndex }

::= { pktClassTable 1 }

PktClassEntry ::= SEQUENCE {

pktClassIndex			INTEGER,

pktClassDscpLow         INTEGER,

pktClassDscpHigh        INTEGER,

pktClassDscpMarkValue   INTEGER,

pktClassIPProtocol      INTEGER,

pktClassIPSrcAddr       IpAddress,

pktClassIPSrcAddrMask   IpAddress,

pktClassIPDstAddr       IpAddress,

pktClassIPDstAddrMask   IpAddress,

pktClassSrcPortLow      INTEGER,

pktClassSrcPortHigh     INTEGER,

pktClassDstPortLow      INTEGER,

pktClassDstPortHigh     INTEGER,

pktClassDstMacAddr      PhysAddress,

pktClassDstMacMask      PhysAddress,

pktClassSrcMacAddr      PhysAddress,

pktClassVlanId          INTEGER,

pktClassVlanCoS         INTEGER,

pktClassPhbAssociation  INTEGER,

pktClassStatus			INTEGER

}

pktClassIndex    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index"

::={pktClassEntry 1}

pktClassDscpLow OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the low value of a range

of DSCP values to which a packet is compared. "

::={pktClassEntry 2}

pktClassDscpHigh OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the high value of a range of

DSCP values to which a packet is compared."

::={pktClassEntry 3}

pktClassDscpMarkValue OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object is the DSCP value used to mark the packet,

-1 indicates no DSCP marking, possible DSCP marks values

are (0..63)"

::={pktClassEntry 4}

pktClassIPProtocol OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the IP protocol to which a

packet is compared."

::={pktClassEntry 5}

pktClassIPSrcAddr     OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the IP source address to which a

packet is compared."

::={pktClassEntry 6}

pktClassIPSrcAddrMask  OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP source

address will be matched."

::={pktClassEntry 7}

pktClassIPDstAddr      OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the IP destination address to which a

packet is compared."

::={pktClassEntry 8}

pktClassIPDstAddrMask  OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP destination

address will be matched."

::={pktClassEntry 9}

pktClassSrcPortLow OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the low range of the source

port to which a packet is compared. "

::={pktClassEntry 10}

pktClassSrcPortHigh  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the high range of the source

port to which a packet is compared. "

::={pktClassEntry 11}

pktClassDstPortLow   OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the low range of the destination

port to which a packet is compared. "

::={pktClassEntry 12}

pktClassDstPortHigh  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the high range of the destination

port to which a packet is compared. "

::={pktClassEntry 13}

pktClassDstMacAddr  OBJECT-TYPE

SYNTAX              PhysAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the destination MAC address to

which a packet is compared. "

::={pktClassEntry 14}

pktClassDstMacMask  OBJECT-TYPE

SYNTAX              PhysAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the mask of the destination MAC address

to which a packet is compared. "

::={pktClassEntry 15}

pktClassSrcMacAddr        OBJECT-TYPE

SYNTAX              PhysAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the source MAC address to which a packet

is compared."

::={pktClassEntry 16}

pktClassVlanId  OBJECT-TYPE

SYNTAX              INTEGER (0..7)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the VLAN id to which a packet is compared."

::={pktClassEntry 17}

pktClassVlanCoS OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object specifies the VLAN Class of Service (CoS)

to which a packet is compared. "

::={pktClassEntry 18}

pktClassPhbAssociation OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Associate the filter entry to a specific PHB (refer to PhbIndex)."

::={pktClassEntry 19}

pktClassStatus   OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

::={pktClassEntry 20}

-----PhbMappingTable

phbMappingTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF PhbMappingEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of PHB MIB entries. This class describes the PHB mapping with the Request Class. The number of entries is specified by PhbIndex."

::={qos 2}

phbMappingEntry        OBJECT-TYPE

SYNTAX            PhbMappingEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the PHB mapping table."

INDEX   { phbIndex }

::= { phbMappingTable 1 }

PhbMappingEntry ::= SEQUENCE {

phbIndex					INTEGER,

phbIdentifier       		INTEGER,

phbName             		DisplayString,

phbRequestClassAssociation  INTEGER,

phbMappingRowStatus         INTEGER

}

phbIndex    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the PHB mapping table. A maximum of 64 entries can be created."

::={phbMappingEntry 1}

phbIdentifier OBJECT-TYPE

SYNTAX              INTEGER (0..63)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Identification of the PHB (0..63)."

::={phbMappingEntry 2}

phbName  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Name of the PHB"

::={phbMappingEntry 3}

phbRequestClassAssociation  OBJECT-TYPE

SYNTAX               INTEGER (0..15)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object is an association with this PHB

and a Request class entry in the Request class table.

(refer to a Request Class index)"

::={phbMappingEntry 4}

phbMappingRowStatus   OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={phbMappingEntry 5}

----qos.RequestClassTable

requestClassTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF RequestClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of Request class entries. This class describes the layer 2 QoS objects. The number of entries is specified by RequestClassIndex. "

::={qos 3}

requestClassEntry        OBJECT-TYPE

SYNTAX            RequestClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Request class table."

INDEX   { requestClassIndex,requestClassPidIndex }

::= { requestClassTable 1 }

RequestClassEntry ::= SEQUENCE {

requestClassIndex			INTEGER,

requestClassName            DisplayString,

requestClassChanId          INTEGER,

requestClassVccVpi          INTEGER,

requestClassVccVci          INTEGER,

requestClassPidIndex		INTEGER,

requestClassPidValue    	INTEGER,

requestClassCra             INTEGER,

requestClassRbdcMax         INTEGER,

requestClassRbdcTimeout     INTEGER,

requestClassVbdcMax         INTEGER,

requestClassVbdcTimeout     INTEGER,

requestClassVbdcMaxBackLog  INTEGER,

requestClassAvbdcRepTime    INTEGER,

requestClassRowStatus       INTEGER

}

requestClassIndex    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the Request Class table. A maximum of 16 entries can be created."

::={requestClassEntry 1}

requestClassName   OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Name of the Request class."

::={requestClassEntry 2}

requestClassChanId  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Channel id of the Request class."

::={requestClassEntry 3}

requestClassVccVpi OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Defines VPI used for the Request class (ATM profile)."

::={requestClassEntry 4}

requestClassVccVci  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Defines VCI used for the Request class (ATM profile)."

::={requestClassEntry 5}

requestClassPidIndex OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the Request class Pid table. Only one Pid entry is

required for short term QoS support "

::={requestClassEntry 6}

requestClassPidValue OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Define the PID used for the Request Class.

One PID per RC will be used for the short term QoS support.  "

::={requestClassEntry 7}

requestClassCra  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Define CRA level for the Request class in bit per second (b/s)."

::={requestClassEntry 8}

requestClassRbdcMax OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Max RBDC that can be requested for the Request class, in number of 2 Kb/s"

::={requestClassEntry 9}

requestClassRbdcTimeout OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Persistence of the RBDC request, expressed in superframes"

::={requestClassEntry 10}

requestClassVbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Max VBDC that can be requested for the Request class,

in payload units (one ATM cell or one MPEG packet) per superframe"

::={requestClassEntry 11}

requestClassVbdcTimeout OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time after which the RCST considers that the pending requests are lost.

The RCST may issue new request for that traffic. VBDC Timeout is

expressed in superframes."

::={requestClassEntry 12}

requestClassVbdcMaxBackLog OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"VBDC back Log per Request class. (expressed in bytes)"

::={requestClassEntry 13}

requestClassAvbdcRepTime OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Optional parameter. AVBDC requests can be sent periodically,

according to the value of that parameter.

AVBDCRepTime is expressed in superframes."

::={requestClassEntry 14}

requestClassRowStatus  OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={requestClassEntry 15}

globalRbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global RBDC max is for information only and it is optional.

(expressed in number of 2 kb/s)"

::={qos 4}

globalVbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global VBDC max is for information only and it is optional.

(expressed in payload units/superframe)"

::={qos 5}

globalVbdcMaxBackLog OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global VBDC back log is used only if the VBDC back log is not

configured in the Request class.	(expressed in bytes)"

::={qos 6}

globalAvbdcRepTime  OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global AVBDC rep time is optional (expressed in superframes)."

::={qos 7}

channelIdStrictDispatching OBJECT-TYPE

SYNTAX              INTEGER {

not_strict (0),

strict (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Indicates whether the RCST will strictly follow RC association when

signaled through Channel_ID in the TBTP:

No strict association (0)

Strict association (1)"

::={qos 8}

---- rcstStatus group

rcstMode  OBJECT-TYPE

SYNTAX              INTEGER {

installation (0),

operational (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Allows to identify in which mode the RCST is and to return it to

the installation mode when needed. Values for the RCST mode are:

Installation (0)

Operational (1)"

::={rcstStatus 1}

faultStatus  OBJECT-TYPE

SYNTAX              INTEGER {

nofault (0),

fault (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provide the fault status of the terminal. The fault status management

is vendor specific. Values for the Fault Status are:

no fault (0)

fault (1)"

::={rcstStatus 2}

fwdLinkStatus OBJECT-TYPE

SYNTAX              INTEGER {

notAcquired (0),

acquired 	(1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Forward Link. Values for the

Forward Link Status are:

Not acquired (0)

Acquired (1)"

::={rcstStatus 3}

rtnLinkStatus   OBJECT-TYPE

SYNTAX              INTEGER {

loggedOff (0),

loggedOn  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Return Link. Values for the

Return Link Status are:

Logged-off (0)

Logged-on (1)"

::={rcstStatus 4}

logUpdated OBJECT-TYPE

SYNTAX              INTEGER {

noUpdate 		(0),

logfileUpdated  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the existence of an updated log file:

No update (0)

Log file updated (1)

The RCST should remove the Log file updated indication

as the log file is fetched by the NCC."

::={rcstStatus 5}

downloadStatus OBJECT-TYPE

SYNTAX              INTEGER {

success	(1),

failure (2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provide some information about the result of the software

download process. The following values shall be supported:

success (1)

failure (2)"

::={rcstStatus 6}

currentSoftwareVersion  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Current software version of the Software Download material."

::={rcstStatus 7}

alternateSoftwareVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Alternate software version of the Software Download material. "

::={rcstStatus 8}

currentConfigFileVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the activated configuration file. Version is vendor specific"

::={rcstStatus 9}

pendingConfigFileVersion

OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the last downloaded configuration file.

Version is vendor specific"

::={rcstStatus 10}

--- rcstAction group

rebootCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

normal 		(2),

alternate 	(3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to reboot

(1)- idle

(2)- normal reboot (from current software load)

(3)- reboot from alternate load (swap to alternate load before reboot)"

DEFVAL {1}

::={rcstAction 1}

rcstTxDisable	OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

disable 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to stop transmission (transmit disabled as defined in [1]):

(1)- idle

(2)- initiate Tx Disabled"

DEFVAL {1}

::={rcstAction 2}

userTrafficDisable	OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

disable 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall disable user traffic (only RCST management traffic can be transmitted)

(1)- idle

(2)- disable user traffic"

DEFVAL {1}

::={rcstAction 3}

cwEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to start transmission of CW :

(1)- off

(2)- on"

DEFVAL {1}

::={rcstAction 4}

oduTxReferenceEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of 10 MHz reference

clock in the Tx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstAction 5}

oduTxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Tx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstAction 6}

oduRxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Rx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstAction 7}

downloadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle			(1),

config	 		(2),

installationLog	(3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST configuration file download process

(1)	idle

(2)	download RCST configuration file from TFTP/FTP server

(3)	download RCST installation log file from TFTP/FTP server

(INSTALL_LOG OPTION)"

DEFVAL {1}

::={rcstAction 8}

uploadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle			(1),

config 			(2),

eventAlarm 		(3),

installationLog (4)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST upload process

(1)	idle

(2)	upload RCST configuration file to TFTP/FTP server

(3)	upload RCST event/alarm log file to TFTP/FTP server

(4)	upload RCST installation log file to TFTP/FTP server

(INSTALL_LOG OPTION)"

DEFVAL {1}

::={rcstAction 9}

activateConfigFileCommand  OBJECT-TYPE

SYNTAX           INTEGER {

idle			(1),

activate		(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Triggers the RCST to use the configuration file and update

its parameters accordingly.  Some RCST implementation may require

a reboot for the parameters to take effect (vendor specific).

(1)	Idle

(2)	activate"

DEFVAL {1}

::={rcstAction 10}

rcstLogonCommand	OBJECT-TYPE

SYNTAX           INTEGER {

idle	(1),

logon	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST logon

(1) idle

(2) initiate RCST logon"

DEFVAL {1}

::={rcstAction 11}

rcstLogoffCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle	(1),

logoff	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST logoff

(1) idle

(2) initiate RCST logoff"

DEFVAL {1}

::={rcstAction 12}

END=

# Security Considerations

Some of the readable objects in this MIB module (i.e., objects with a MAX-ACCESS other than not-accessible) may be considered sensitive or vulnerable in some network environments.

SNMP versions prior to SNMPv3 did not include adequate security. Even if the network itself is secure (for example by using IPsec), even then, there is no control as to who on the secure network is allowed to access and GET/SET (read/change/create/delete) the objects in this MIB module.

It is RECOMMENDED that implementers consider the security features as provided by the SNMPv3 framework (see [RFC3410], section 8), including full support for the SNMPv3 cryptographic mechanisms (for authentication and privacy).

Further, deployment of SNMP versions prior to SNMPv3 is NOT RECOMMENDED.  Instead, it is RECOMMENDED to deploy SNMPv3 and to enable cryptographic security.  It is then a customer/operator responsibility to ensure that the SNMP entity giving access to an instance of this MIB module, is properly configured to give access to the objects only to those principals (users) that have legitimate rights to indeed GET or SET (change/create/delete) them.

# IANA Considerations

No action from IANA is needed.

# Acknowledgments

The authors thank Gorry Fairhurst for advice in the preparation of this document.

The authors recognize this document is a collective effort of the SatLabs Group (www.satlabs.org).

# References

## Normative References

[RFC2119]	Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997 (BEST CURRENT PRACTICE).

[RFC2578] McCloghrie, K., Ed., Perkins, D., Ed., and J. Schoenwaelder, Ed., "Structure of Management Information, Version 2 (SMIv2)", STD 58, RFC 2578, April 1999.

[RFC2579]  McCloghrie, K., Ed., Perkins, D., Ed., and J.               Schoenwaelder, Ed., "Textual Conventions for SMIv2",               STD 58, RFC 2579, April 1999.

[RFC2580] McCloghrie, K., Perkins, D., and J. Schoenwaelder, "Conformance Statements for SMIv2", STD 58, RFC 2580, April 1999.

## Informative References

[ISO-MPEG] ISO/IEC DIS 13818-1:2000, "Information Technology;                 Generic Coding of Moving Pictures and Associated Audio Information Systems", International Standardisation Organisation (ISO).

[ETSI-DAT] EN 301 192, "Digital Video Broadcasting (DVB); DVB Specifications for Data Broadcasting", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS] EN 301 421 "Digital Video Broadcasting (DVB); Modulation and Coding for DBS satellite systems at 11/12 GHz", European Telecommunications Standards Institute (ETSI).

[ETSI-RCS] ETSI 301 791 "Digital Video Broadcasting (DVB); Interaction Channel for Satellite Distribution Systems", European Telecommunications Standards Institute (ETSI).

[RFC3410] Case, J., Mundy, R., Partain, D., and B. Stewart, "Introduction and Applicability Statements for Internet-Standard Management Framework", RFC 3410, December 2002. [RFC4259] Montpetit, M.-J., Fairhurst, G., Clausen, H., Collini-Nocker, B., and H. Linder, "A Framework for Transmission of IP Datagrams over MPEG-2 Networks", RFC 4259, November 2005.

[SATLABS]	SatLabs System Recommendations, version 2, December 2006. Available at www.satlabs.org.

# Authors' Addresses

Stephane Combes

ESTEC

European Space Agency

Keplerlaan 1 – P.O. Box 299

2200 AG Noordwijk ZH

The Netherlands

Email: stephane.combes@esa.int

URL:   telecom.esa.int

Petter Chr. Amundsen

VeriSat AS

P.O Box 1

1330 Fornebu

Norway

Email: pca@verisat.no

URL:   www.verisat.no

Micheline Lambert

Advantech Satellite Networks

2341 boul. Alfred-Nobel

Saint-Laurent (Montreal,)H4S 2A9

Quebec, Canada

Email: micheline.lambert@advantechamt.com

URL:   www.advantechsatnet.com

Hans-Peter Lexow

STM Norway

Vollsveien 21

1366 Lysaker

Norway

Email: hlexow@stmi.com

URL:   www.stmi.com

# Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any Intellectual Property Rights or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; nor does it represent that it has made any independent effort to identify any such rights.  Information on the procedures with respect to rights in RFC documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the IETF on-line IPR repository at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights that may cover technology that may be required to implement this standard.  Please address the information to the IETF at ietf-ipr@ietf.org.

# Disclaimer of Validity

This document and the information contained herein are provided on an "AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY, THE IETF TRUST AND THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF  THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

# Copyright Statement

Copyright (C) The IETF Trust (2007).

This document is subject to the rights, licenses and restrictions

contained in BCP 78, and except as set forth therein, the authors  retain all their rights.

<<< RFC Ed Note: please remove the text below, prior to publications >>>

Change Log.

Rev -00

- First draft, for comment by the community.

<<<End of RFC Ed Note >>