| Internet Engineering Task Force | Gorry Fairhurst |
| --- | --- |
| Internet Draft | University of Aberdeen, U.K. |
| Document: draft-ietf-ipdvb-ule-04.txt | Bernhard Collini-Nocker |
|  | University of Salzburg, A |
| ipdvb WG |  |
| Category: Draft, Intended Standards Track | January 2005 |

- Ultra Lightweight Encapsulation (ULE) for transmission of

IP datagrams over MPEG-2/DVB networks

- Status of this Draft

By submitting this Internet-Draft, each author represents that any applicable patent or other IPR claims of which he or she is aware have been or will be disclosed, and any of which he or she becomes aware will be disclosed, in accordance with Section 6 of RFC 3668.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups. Note that other groups may also distribute working documents as Internet- Drafts. Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

The list of current Internet-Drafts can be accessed at http://www.ietf.org/1id-abstracts.html

The list of Internet-Draft Shadow Directories can be accessed at http://www.ietf.org/shadow.html.

Abstract

The MPEG-2 TS has been widely accepted not only for providing digital TV services, but also as a subnetwork technology for building IP networks. This document describes an Ultra Lightweight Encapsulation (ULE) mechanism for the transport of IPv4 and IPv6 Datagrams and other network protocol packets directly over ISO MPEG-2 Transport Streams (TS) as TS Private Data. ULE supports an extension format that allows it to carry both optional (with an explicit extension length) and mandatory (with an implicit extension length) header information to assist in network/Receiver processing of a SNDU.

[RFC EDITOR NOTE:

This section must be deleted prior to publication]

DOCUMENT HISTORY

Draft 00

This draft is intended as a study item for proposed future work by the IETF in this area.  Comments relating to this document will be gratefully received by the author(s) and the ip-dvb mailing list at: ip-dvb@erg.abdn.ac.uk

--------------------------------------------------------------------

DRAFT 01 (Protocol update)

* Padding sequence modified to 0xFFFF, this change aligns with other usage by MPEG-2 streams. Treatment remains the same as specified for ULE.

* SDNU Format updated to include R-bit (reserved).

* Procedure for TS Packet carrying the final part of a SNDU with either less than two bytes of unused payload updated.

* A Receiver MUST silently discard the remainder of a TS Packet payload when two or less bytes remain unprocessed following the end of a SNDU, irrespective of the PUSI value in the received TS Packet. It MUST NOT record an error when the value of the remaining byte(s) is identical to 0xFF or 0xFFFF.  The Receiver MUST then wait for a TS Packet with a PUSI value set to 1.

* Payload Pointer description updated.

* CRC Calculation added.

* Decapsulator processing revised.

* Type field split into two.

* References updated.

* Security considerations added (first draft).

* Appendix added with examples.

--------------------------------------------------------------------

DRAFT - 02 (Improvement of clarity)

* Corrected CRC-32 to follow standard practice in DSM-CC.

* Removed LLC frame type, now redundant by Bridge-Type (==1)

* Defined D-bit to use the reserved bit field (R ) - Gorry, Alain, Bernhard

* Changes to description of minimum payload length. Gorry

* MPEG-2 Error Indicator SHOULD be used.Hilmar & Gorry

* MPEG-2 CC MAY be used (since CRC-32 is strong anyway). Hilmar & Gorry

* Corrected CRC-32 to now follow standard practice in DSM-CC. Gorry, Hilmar, Alain.

* Changed description of Encapsulator action for Packing. Gorry & Hilmar.

* Changed description of Receiver to clarify packing. Gorry & Alain.

* Stuff/Pad of unused bytes MUST be 0xFF, to align with MPEG. Hilmar/Bernhard.

* Recommend removal of section on Flushing bit stream. Gorry

* Updated SNDU figures to reflect D-bit and correct a mistake in the bridged type field. Alain

* Reorganised section 5 to form sections 5 and 6, separating encapsulation and receiver processing. Gorry, Hilmar, Alain.

* Added concept of Idle State and Reassembly State to the Receiver.

Renumbered sections 5,6 and following.  Gorry.

* Nits from Alain, Hilmar and Gorry.

Moved security issue on the design of the protocol to appropriate sections, since this is not a concern for deployment: Length field usage and padding initialisation.

* Changed wording: All multi-byte values in ULE (including Length, Type, and Destination fields) are transmitted in network byte order (most significant byte first). old NiT from Alain, now fixed.

* Frame byte size in diagrams now updated to -standard- format, and D bit action corrected, as requested by Alain.

* Frame format diagrams, redrawn to 32-bit format below:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

* Additional diagram requested by Alain for D=0 bridging (added, and subsequent figures renumbered).

* Diagrams of encapsulation process, redrawn for clarity (no change to meaning). Gorry.

* Reworded last para of CRC description.

* Clarification to the statements in the CRC coverage - to make it clear that it is the entire SNDU (header AND payload) that is checksummed. (Fritsche@iabg.de, hlinder@cosy.sbg.ac.at).

* References added for RCS (spotted by Alain) and AAL5 (provided by Anthony Ang).

* Removed informative reference to MPEG part 1.Alain.

Spelling correction -> Allain to Alain.

* Added description of Receiver processing of the address field.Gorry

* Added caution on LLC Length in bridged Packets thanks. Gorry/wolfgang

* Removed Authors notes from text after their discussion on the list Gorry

* Corrected text to now say maximum value of PP = 182 in ULE. Gorry

* Tidied diagrams at end (again) - Gorry,

Revision with following changes:

* Re issue as working group draft (filename change)

* Refinement of the text on CRC generation to be unambiguous.

* Revised CC processing at Encapsulator (B C-N/GF/A.Allison)

* Revised CC processing at Receiver (from List: A.Allison; et al )

* Corrections to length/PP field in Examples (M Sooriyabandara, Alain)

* Corrections to pointer in Example 3 SNDU C (M Jose-Montpetit)

* Section 4.5 only SHARED routed links require D=0

* Packing Threshold defined

* Next-Layer-Header defined (Now called Next-Header)

* Addition of Appendix B (to aide verification of SNDFU format)

Working Group ID rev 01

Issues addressed:

* Typographical

* Types > 1500 should be passed to the next higher protocol (Hilmar)

* The second part of the Type space corresponds to the values 1500 COMMENT: ~Range should be 1536 Decimal Decimal to 0xFFFF.

* IANA has already defined IP and IPv6 types - corrected text!

Added more security considerations (-01d).

* Should we allow an Adaptation Field within ULE (request for DVB-RCS compatibility)? Requirement to be clarified! Implementation impact to be evaluated!

Current Recommendation: The current spec does not preclude use of AF, it simply says that this is not the standard for ULE.  The use case and requirement for this mode are not currently clear, based on this there is no current intention to add this to ULE - text for requirements would be welcome.

* Verify the minimum value allocated to DIX Ethernet Header Types.

Draft updated to align with IEEE Registry assignments.

--------------------------------------------------------------------

Working Group ID rev 02

Revised IPR disclosure

Revised copyright notice

Section 5 added to ULE to define optional Extension Headers (see xule)

Correction of figure numbering.

Correction to capitalisation in Transport Stream definition of fields

Inserted space character after 1536 in line 2 of 4.4.2

Replaced } with ] after ISO_DSMCC

Replace reference to section 6.3 with section 7.3 at end of section 4.6.

Reference in 4.7.4 was changed to refer to figure 7 (not 6).

Note added after figure 9.

Working Group ID rev 03

Changes with this revision of the document:

- The worked hexadecimal example in the annexe was reworked to include a valid MAC address for an IPv6 unicast packet. - (BCN)

- The IANA procedures revised, based on inputs from IANA to improve consistency of the term Next-Header and to add the HLEN field to the IANA registry record for OPTIONAL headers. (GF)

- 7.2 Change to revert wording in the second para to MUST enter IDLE after CRC failure of SNDU check.

- In normal operation, it is expected that any padding appended to a bridged Ethernet frame SHOULD be removed prior to forwarding. This requires the sender to be aware of such Ethernet padding (e.g. LLC). (Made this a SHOULD). (GF)

NiTS:

- Format of page Breaks was updated. (GF)

- Check for <- -> sequences of characters. (GF)

- Update refs to add RFC3667 / 3668. (GF)

- Changed text defining M in DSMCC definition to the word Media

- 7.1.1 Range of PP values corrected to 0-181.

- Definition of END INDICATOR corrected in section 2 - this is not a TYPE value, but a LENGTH value.

- Next-Header used throughout the document to replace

next-layer-header, and various other forms of wording.

- In section 7.2, added a ref the section on PP checking

[END of RFC EDITOR NOTE]

Table of Contents

1. Introduction

2. Conventions used in this document

3. Description of method

4. SNDU Format

4.1 Destination Address Present Field

4.2 Length Field

4.3 End Indicator

4.4 Type Field

4.4.1 Type 1: Next-Header Type Fields

4.4.2 Type 2: EtherType Compatible Type Fields

4.5 SNDU Destination Address Field

4.6 SNDU Trailer CRC

4.7 Description of SNDU Formats

4.7.1 End Indicator

4.7.2 IPv4 SNDU Encapsulation

4.7.3 IPv6 SNDU Encapsulation

5. Extension Headers

5.1 Test SNDU

5.2 Bridged SNDU

6.Processing at the Encapsulator

6.1 SNDU Encapsulation

6.2 Procedure for Padding and Packing

7. Receiver Processing

7.1 Idle State

7.1.1 Reassembly Payload Pointer Checking

7.2 Processing of a Received SNDU

7.2.1 Reassembly Payload Pointer Checking

7.3 Other Error Conditions

8. Summary

9. Acknowledgments

10. Security Considerations

11. References

11.1 Normative References

11.2 Informative References

12. Authors' Addresses

13. IPR Notices

14. Copyright Statement

14.1     Intellectual Property Statement

14.2 Disclaimer of Validity

15. IANA Considerations

ANNEXE A: Informative Appendix - SNDU Packing Examples

ANNEXE B: Informative Appendix - SNDU Encapsulation

1. Introduction

This document describes an encapsulation for transport of IP datagrams, or other network layer packets, over ISO MPEG-2 Transport Streams [ISO-MPEG; ID-ipdvb-arch].  It is suited to services based on MPEG-2, for example the Digital Video Broadcast (DVB) architecture, the Advanced Television Systems Committee (ATSC) system [ATSC; ATSC-G], and other similar MPEG-2 based transmission systems. Such systems provide unidirectional (simplex) physical and link layer standards.  Support has been defined for a wide range of physical media (e.g. Terrestrial TV [ETSI-DVBT; ATSC-PSIP-TC], Satellite TV [ETSI-DVBS; ATSC-S], Cable Transmission [ETSI-DVBC; ATSC-PSIP-TC]). Bi-directional (duplex) links may also be established using these standards (e.g., DVB defines a range of return channel technologies, including the use of two-way satellite links [ETSI-RCS] and dial-up modem links [RFC3077]).

Protocol Data Units, PDUs, (Ethernet Frames, IP datagrams or other network layer packets) for transmission over an MPEG-2 Transport Multiplex are passed to an Encapsulator. This formats each PDU into a Subnetwork Data Unit (SNDU) by adding an encapsulation header and an integrity check trailer. The SNDU is fragmented into a series of TS Packets) that are sent over a single TS Logical Channel.

2. Conventions used in this document

ADAPTATION FIELD: An optional variable-length extension field of the fixed-length TS Packet header, intended to convey clock references and timing and synchronization information as well as stuffing over an MPEG-2 Multiplex [ISO-MPEG].

AFC: Adaptation Field Control, a pair of bits carried in the TS Packet header that signal the presence of the Adaptation Field and/or TS Packet payload.

ATSC: Advanced Television Systems Committee [ATSC]. A framework and a set of associated standards for the transmission of video, audio, and data using the ISO MPEG-2 standard.

DSM-CC: Digital Storage Media Command and Control [ISO-DSMCC]. A format for transmission of data and control information defined by the ISO MPEG-2 standard that is carried in an MPEG-2 Private Section.

DVB: Digital Video Broadcast [ETSI-DVB]. A framework and set of associated standards published by the European Telecommunications Standards Institute (ETSI) for the transmission of video, audio, and data, using the ISO MPEG-2 Standard.

ENCAPSULATOR: A network device that receives PDUs and formats these into Payload Units (known here as SNDUs) for output as a stream of TS Packets.

END INDICATOR: A value that indicates to the Receiver that there are no further SNDUs present within the current TS Packet.

MAC: Medium Access and Control. The link layer header of the Ethernet IEEE 802 standard of protocols, consisting of a 6B destination address, 6B source address, and 2B type field.

MPE: Multiprotocol Encapsulation [ETSI-DAT; ATSC-DAT ; ATSC-DATG]. A scheme that encapsulates PDUs, forming a DSM-CC Table Section. Each Section is sent in a series of TS Packets using a single TS Logical Channel.

MPEG-2: A set of standards specified by the Motion Picture Experts Group (MPEG), and standardized by the International Standards Organisation (ISO) [ISO-MPEG].

NEXT-HEADER: A Type value indicating an Extension Header.

NPA: Network Point of Attachment. In this document, refers to a 6 B destination address (similar to an Ethernet MAC address) within the MPEG-2 transmission network used to identify individual Receivers or groups of Receivers.

PACKING THRESHOLD: A period of time an Encapsulator is willing to defer transmission of a partially filled TS-Packet to accumulate more SNDUs, rather than use Padding. After the Packet Threshold period, the Encapsulator uses Padding to send the partially filled TS-Packet.

PDU: Protocol Data Unit. Examples of PDU include Ethernet frames, IPv4 or IPv6 datagrams, and other network packets

PES: Packetized Elementary Stream of MPEG-2 [ISO-MPEG].

PID: Packet Identifier. A 13 bit field carried in the header of TS Packets. This is used to identify the TS Logical Channel to which a TS Packet belongs [ISO-MPEG]. The TS Packets forming the parts of a Table Section, PES, or other payload unit must all carry the same PID value.  The all 1s PID value indicates a Null TS Packet introduced to maintain a constant bit rate of a TS Multiplex.

PP: Payload Pointer. An optional one byte pointer that directly follows the TS Packet header. It contains the number of bytes between the end of the TS Packet header and the start of a Payload Unit. The presence of the Payload Pointer is indicated by the value of the PUSI bit in the TS Packet header. The Payload Pointer is present in DSM-CC, and Table Sections, it is not present in TS Logical Channels that use the PES-format.

PU: Payload Unit. A sequence of bytes sent using a TS. Examples of Payload Units include: an MPEG-2 Table Section or a ULE SNDU.

PUSI: Payload_Unit_Start_Indicator of MPEG-2 [ISO-MPEG]. A single bit flag carried in the TS Packet header. A PUSI value of zero indicates that the TS Packet does not carry the start of a new Payload Unit. A PUSI value of one indicates that the TS Packet does carry the start of a new Payload Unit. In ULE, a PUSI bit set to 1 also indicates the presence of a one byte Payload Pointer (PP).

PRIVATE SECTION: a syntactic structure used for mapping all service information (e.g. an SI table) into TS Packets.  A Table may be divided into a number of Table Sections, however all Table Sections must be carried over a single TS Logical Channel.

PSI: Program Specific Information. Tables used to convey information about the service carried in a TS Multiplex. The set of PSI tables is defined by [ISO-MPEG], see also SI Table.

SI TABLE: Service Information Table. In this document, this term describes any table used to convey information about the service carried in a TS Multiplex. SI tables are carried in MPEG-2 private sections.

SNDU: Subnetwork Data Unit. An encapsulated PDU sent as an MPEG-2 Payload Unit.

TABLE SECTION: A Payload Unit carrying a part of a MPEG-2 SI Table.

TS: Transport Stream [ISO-MPEG], a method of transmission at the MPEG-2 level using TS Packets; it represents level 2 of the ISO/OSI reference model. See also TS Logical Channel and TS Multiplex.

TS HEADER: The 4 byte header of a TS Packet as illustrated in the introduction.

TS LOGICAL CHANNEL: Transport Stream Logical Channel, a channel identified at the MPEG-2 level [ISO-MPEG]. It exists at level 2 of the ISO/OSI reference model. All packets sent over a TS Logical Channel carry the same PID value. According to MPEG-2, some TS Logical Channels are reserved for specific signalling purposes. Other standards (e.g., ATSC, DVB) also reserve specific TS Logical Channels.

TS MULTIPLEX: A set of MPEG-2 TS Logical Channels sent over a single common physical link (i.e. a transmission at a specified symbol rate, FEC setting, and transmission frequency). The same TS Logical Channel may be repeated over more than one TS Multiplex, for example to redistribute the same multicast content to two terrestrial TV transmission cells.

TS PACKET: A fixed-length 188B unit of data sent over a TS Multiplex [ISO-MPEG]. Each TS Packet carries a 4B header, plus optional overhead including an Adaptation Field, encryption details and time stamp information to synchronise a set of related Transport Streams. The 188B TS Packets incorporate a 4B header with the following fields (those referenced within this document are marked with *):

Field Length            Name/Purpose

(in bits)

8b             Synchronisation pattern equal 0x47

*1b             Transport Error Indicator

*1b             Payload Unit Start Indicator (PUSI)

1b             Transport Priority

*13b            Packet IDentifier (PID)

2b             Transport scrambling control

*2b             Adaptation Field Control (AFC)

*4b             Continuity Counter (CC)

3. Description of the Method

PDUs (IP packets, Ethernet frames or packets from other network protocols) are encapsulated to form a Subnetwork Data Unit (SNDU). The SNDU is transmitted over an MPEG-2 transmission network by placing it either in the payload of a single TS Packet, or if required, an SNDU may be fragmented into a series of TS Packets. Where there is sufficient space, the method permits a single TS Packet to carry more than one SNDU (or part there of), sometimes known as Packing. All TS Packets comprising a SNDU MUST be assigned the same PID, and therefore form a part of the same TS Logical Channel.

The ULE encapsulation is limited to TS private streams only. The header of each TS Packet carries a one bit Payload Unit Start Indicator (PUSI) field. The PUSI identifies the start of a payload unit (SNDU) within the MPEG-2 TS Packet payload. The semantics of the PUSI bit are defined for PES and PSI packets [ISO-MPEG]; for private data, its use is not defined in the MPEG-2 Standard. In ULE, although being private data, the operation follows that of PSI packets. Hence, the following PUSI values are defined:

0: The TS Packet does NOT contain the start of a SNDU, but contains the continuation, or end of a SNDU;

1: The TS Packet contains the start of a SNDU, and a one byte Payload Pointer follows the last byte of the TS Packet header.

If a Payload Unit (SNDU) finishes before the end of a TS Packet payload, but it is not intended to start another Payload Unit, a stuffing procedure fills the remainder of the TS Packet payload with bytes with a value 0xFF [ISO-MPEG2], known as Padding.

A Receiver processing MPEG-2 Table Sections is aware that when it receives a table_id value of 0xFF, this indicates Padding/Stuffing occurred and silently discards the remainder of the TS Packet payload. The payload of the next TS Packet for the same TS Logical Channel will begin with a Payload Pointer of value 0x00, indicating that the next Payload Unit immediately follows the TS Packet header. The ULE protocol resembles this, but differs in the exact procedure (see the following sections).

The TS Packet Header also carries a two bit Adaptation Field Control (AFC) value. The purpose of the adaptation field is primarily to extend the TS header for timing and synchronisation information and may be used to also include stuffing bytes before a TS Packet payload. Standard Receivers discard TS Packets with an adaptation_field_control field value of '00'. Adaptation Field stuffing is NOT used in this encapsulation method, and TS Packets from a ULE Encapsulator MUST be sent with an AFC value of '01'. Receivers MUST discard TS Packets that carry other AFC values.

4. SNDU Format

PDUs (IP packets and bridged Ethernet frames) are encapsulated using ULE to form a SNDU. Each SNDU is sent as an MPEG-2 Payload Unit. The encapsulation format to be used for PDUs is illustrated below:

< ----------------------------- SNDU ----------------------------- >

+-+-------------------------------------------------------+--------+
|D| Length | Type |                 PDU                   | CRC-32 |
+-+-------------------------------------------------------+--------+

Figure 1: SNDU Encapsulation

All multi-byte values in ULE (including Length, Type, and Destination fields) are transmitted in network byte order (most significant byte first). Appendix A provides informative examples of usage.

4.1 The Destination Address Present Field

The most significant bit of the Length Field carries the value of the Destination Address Present Field, the D-bit. A value of 0 indicates the presence of the Destination Address Field (see section 4.5). A value of 1 indicates that a Destination Address Field is not present (i.e. it is omitted).

By default, the D-bit value MUST be set to a value of 0, except for the transmission of an End Indicator (see 4.3), in which this bit MUST be set to the value of 1.

4.2 Length Field

A 15-bit value that indicates the length, in bytes, of the SNDU (encapsulated Ethernet frame, IP datagram or other packet) counted from the byte following the Type field, up to and including the CRC. Note the special case described in 4.3.

4.3 End Indicator

When the first two bytes of a SNDU have the value 0xFFFF, this denotes an End Indicator (i.e., all 1s length combined with a D-bit value of 1). It indicates to the Receiver that there are no further SNDUs present within the current TS Packet (see section 6), and that no Destination Address Field is present. The value 0xFF has specific semantics in MPEG-2 framing, where it is used to indicate the presence of Padding. This use resembles [ISO-DSMCC].

4.4 Type Field

The 16-bit Type field indicates the type of payload carried in a SNDU, or the presence of a Next-Header. The set of values that may be assigned to this field is divided into two parts, similar to the allocations for Ethernet.

EtherTypes were originally specified by Xerox under the DIX framework for Ethernet. After specification of IEEE 802.3 [LLC], the set of EtherTypes less than 1536 (0x0600), assumed the role of a length indicator. Ethernet receivers use this feature to discriminate LLC format frames. Hence any IEEE EtherType < 1536 indicates an LLC frame, and the actual value indicates the length of the LLC frame.

There is a potential ambiguous case when a Receiver receives a PDU with two length fields:  The Receiver would need to validate the actual length and the Length field and ensure that inconsistent values are not propagated by the network. Specification of two independent length fields is therefore undesirable.  In the ULE header, this is avoided in the SNDU header by including only one length value, but bridging of LLC frames re-introduces this consideration (section 4.7.5).

The Ethernet LLC mode of identification is not required in ULE, since the SNDU format always carries an explicit Length Field, and therefore the procedure in ULE is modified, as below:

The first set of ULE Type field values comprise the set of values <  1536.  These Type field values are IANA assigned (see 4.4.1), and indicate the Next-Header.

The second set of ULE Type field values comprise the set of values >= 1536. In ULE, this indicates that the value is identical to the corresponding type codes specified by the IEEE/DIX type assignments for Ethernet and recorded in the IANA EtherType registry.

4.4.1 Type 1: Next-Header Type Fields

The first part of the Type space corresponds to the values 0 to 1535 Decimal. These values may be used to identify link-specific protocols and/or to indicate the presence of Extension Headers that carry additional optional protocol fields (e.g. a bridging encapsulation). Use of these values is co-ordinated by an IANA registry.

The following types are defined in this document:

[XXX IANA ACTION REQUIRED XXX]

0x0000: Test SNDU, discarded by the Receiver.

0x0001: Bridged Ethernet Frame (i.e. MAC source address follows)

0x0100: Padding, ignored by the Receiver.

[XXX END OF IANA ACTION REQUIRED XXX]

The remaining values within the first part of the Type space are reserved for Next-Header values allocated by the IANA.

4.4.2 Type 2: EtherType compatible Type Fields

The second part of the Type space corresponds to the values 1536 Decimal (0x600) and 0xFFFF.  This set of type assignments follow DIX/IEEE assignments (but exclude use of this field as a frame length indicator) [LLC].  All assignments in this space MUST use the values defined for IANA EtherType, the following two Type values are used as examples (taken from the IANA EtherTypes registry):

0x0800 : IPv4 Payload

0x86DD : IPv6 Payload

4.5 SNDU Destination Address Field

The SNDU Destination Address Field is optional (see section 4.1). This field MUST be carried (i.e. D=0) for IP unicast packets destined to routers that are sent using shared links (i.e., where the same link connects multiple Receivers). A sender MAY omit this field (D=1) for an IP unicast packet and/or multicast packets delivered to Receivers that are able to utilise a discriminator field (e.g. the IPv4/IPv6 destination address), which in combination with the PID value, could be interpreted as a Link-Level address.

When the SNDU header indicates the presence of a SNDU Destination Address field (i.e. D=0), a Network Point of Attachment, NPA, field directly follows the SNDU Type Field.  NPA destination addresses are 6 Byte numbers, normally expressed in hexadecimal, used to identify the Receiver(s) in a MPEG-2 transmission network that should process a received SNDU. The value 0x00:00:00:00:00:00, MUST NOT be used as a destination address in a SNDU. The least significant bit of the first byte of the address is set to 1 for multicast frames, and the remaining bytes specify the link layer multicast address. The specific value 0xFF:FF:FF:FF:FF:FF is the link broadcast address, indicating this SNDU is to be delivered to all Receivers.

4.6 SNDU Trailer CRC

Each SNDU MUST carry a 32-bit CRC field in the last four bytes of the SNDU. This position eases CRC computation by hardware.  The CRC-32 polynomial is to be used. Examples where this polynomial is also employed include Ethernet, DSM-CC section syntax [ISO-DSMCC] and AAL5 [ITU3563]. This is a 32 bit value calculated according to the generator polynomial represented 0x104C11DB7 in hexadecimal:

x^32+x^26+x^23+x^22+x^16+x^12+x^11+x^10+x^8+x^7+x^5+x^4+x^2+x^1+x^0.

The Encapsulator initialises the CRC-32 accumulator register to the value 0xFFFF FFFF.  It then accumulates a transmit value for the CRC32 that includes all bytes from the start of the SNDU header to the end of the SNDU (excluding the 32-bit trailer holding the CRC-32), and places this in the CRC Field. In ULE, the bytes are processed in order of increasing position within the SNDU, the order of processing bits is NOT reversed.  This use resembles, but is different to that in SCTP [RFC3309].

The Receiver performs an integrity check by independently calculating the same CRC value and comparing this with the transmitted value in the SNDU trailer. SNDUs that do not have a valid CRC, are discarded, causing the Receiver to enter the Idle State.

This description may be suited for hardware implementation, but this document does not imply any specific implementation.  Software-based table-lookup or hardware-assisted software-based implementations are also possible. Annexe B provides an example of an Encapsulated PDU that includes the computed CRC-32 value.

The primary purpose of this CRC is to protect the SNDU (header, and payload) from undetected reassembly errors and errors introduced by unexpected software / hardware operation while the SNDU is in transit across the MPEG-2 subnetwork and during processing at the encapsulation gateway and/or the Receiver. It may also detect the presence of uncorrected errors from the physical link (however, these may also be detected by other means, e.g. section 7.3).

4.7 Description of SNDU Formats

The format of a SNDU is determined by the combination of the Destination Address Present bit (D) and the SNDU Type Field.  The simplest encapsulation places a PDU directly into a SNDU payload.  Some Type 1 encapsulations may require additional header fields. These are inserted in the SNDU directly preceding the PDU.

The following SNDU Formats are defined here:

End Indicator: The Receiver should enter the Idle State.
IPv4 SNDU: The payload is a complete IPv4 datagram

IPv6 SNDU: The payload is a complete IPv6 datagram.
Test SNDU: The payload will be discarded by the Receiver.

Bridged SNDU: The payload carries a bridged MAC or LLC frame.

All other formats are currently reserved.

4.7.1 End Indicator

The format of the End Indicator is shown in figure 2. This format MUST carry a D-bit value of 1.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1|                            0x7FFF                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

=        Arbitrary number (>= 0) bytes with value 0xFF          =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2: SNDU Format for an End Indicator.

4.7.2 IPv4 SNDU

IPv4 datagrams are transported using one of the two standard SNDU structures, in which the PDU is placed directly in the SNDU payload.  The two encapsulations are shown in figures 3 and 4. (Note that in this, and the following figures, the IP datagram payload is of variable size, and is directly followed by the CRC-32).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0|        Length  (15b)        |         Type = 0x0800         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               Receiver Destination Address  (6B)              |

+                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               +

|                                                               |

=                           IPv4 datagram                       =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 3: SNDU Format for an IPv4 Datagram using L2 filtering (D=0).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1|        Length  (15b)        |         Type = 0x0800         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

=                           IPv4 datagram                       =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 4: SNDU Format for an IPv4 Datagram using L3 filtering (D=1).

4.7.3 IPv6 SNDU Encapsulation

IPv6 datagrams are transported using one of the two standard SNDU structures, in which the PDU is placed directly in the SNDU payload.  The two encapsulations are shown in figures 5 and 6.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0|        Length  (15b)        |         Type = 0x086DD        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               Receiver Destination Address  (6B)              |

+                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               +

|                                                               |

=                           IPv6 datagram                       =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 5: SNDU Format for an IPv6 Datagram using L2 filtering (D=0).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1|        Length  (15b)        |         Type = 0x086DD        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

=                           IPv6 datagram                       =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 6: SNDU Format for an IPv6 Datagram using L3 filtering (D=1)

5. Extension Headers

This section describes an extension format for the ULE encapsulation. In ULE, a Type field value less than 1536 Decimal indicates an Extension Header. These values are assigned from a separate IANA registry defined for ULE.

The use of a single Type/Next-Header field simplifies processing and eliminates the need to maintain multiple IANA registries. The cost is that each Extension Header requires at least 2 bytes. This is justified, on the basis of simplified processing and maintaining a simple lightweight header for the common case when no extensions are present.

A ULE Extension Header is identified by a 16-bit value in the Type field. It is organised as a 5-bit zero prefix, a 3-bit H-LEN field and an 8-bit H-Type field, as follows:

0                   1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 0 0 0 0|H-LEN|    H-Type     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 7: Structure of ULE Next-Header Field.

The H-LEN Assignment is described below:

- Indicates a Mandatory Extension Header

- Indicates an Optional Extension Header of length 2B

- Indicates an Optional Extension Header of length 4B

- Indicates an Optional Extension Header of length 6B

- Indicates an Optional Extension Header of length 8B

- Indicates an Optional Extension Header of length 10B

>=6	the combined H-LEN and H-TYPE values indicate the EtherType

of a PDU that directly follows this Type field.

A H-LEN of zero indicates a Mandatory Extension Header. Each Mandatory Extension Header has a pre-defined length that is not communicated in the H-LEN field. No additional limit is placed on the maximum length of a Mandatory Extension Header. A Mandatory Extension Header MAY modify the format or encoding of the enclosed PDU (e.g. to perform encryption and/or compression).

The H-Type is a one byte field that may be either one of 256 Mandatory Header Extensions or one of 256 Optional Header Extensions. The set of currently permitted values for both types of Extension Headers are defined by an IANA Registry (section 15). Registry values for Optional Extensions are specified in the form H=1 (i.e. a decimal number in the range 256-511), but may be used with an H-Length value in the range 1-5.

Two examples of Extension Headers are the Test_SNDU and the use of Extension-Padding. The Test-SNDU Mandatory Extension Header results in the entire PDU being discarded. The Extension-Padding Optional Extension Header results in the following (if any) option header being ignored (i.e. a total of H-LEN 16-bit words).

The general format for an SNDU with Extension Headers is:

< --------------------------   SNDU   ------------------------- >

+---+--------------------------------------------------+--------+

|D=0| Length | T1 | NPA Address | H1 | T2 |    PDU     | CRC-32 |

+---+--------------------------------------------------+--------+

< ULE base header >             <  ext 1  >

Figure 8: SNDU Encapsulation with one Extension Header

Where:

D is the ULE D_bit (in this example D=1, however NPA addresses may

also be used in combination with Extension Headers).

T1 is the base header Type field. In this case, specifying a

Next-Header value.

H1 is a set of fields defined for header type T1.  There may be 0

or more bytes of information for a specific ULE Extension Header.

T2 is the Type field of the next header, or an EtherType > 1535 B

indicating the type of the PDU being carried.

< --------------------------   SNDU   ------------------------- >

+---+---------------------------------------------------+--------+
|D=1| Length | T1 | H1 | T2 | H2 | T3 |       PDU       | CRC-32 |
+---+---------------------------------------------------+--------+

< ULE base header >< ext 1 >< ext 2 >

Figure 9: SNDU Encapsulation with two Extension Headers

Using this method, several Extension Headers MAY be chained in series. Figure 12 shows an SNDU including two Extension Headers. The values of T1 and T2 are both less than 1536 Decimal, each indicates the presence of an Extension Header rather than a directly following PDU. T3 has a value > 1535 indicating the EtherType of the PDU being carried. Although an SNDU may contain an arbitrary number of consecutive Extension Headers, it is not expected that SNDUs will generally carry a large number of extensions.

5.1 Test SNDU

A Test SNDU is of Type 1 (figure 10). The structure of the Data portion of this SNDU is not defined by this document. All Receivers MAY record reception in a log file, but MUST then discard any Test SNDUs. The D-bit MAY be set in a TEST SNDU.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|D|        Length  (15b)        |         Type = 0x0000         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

=               Data (not forwarded by a Receiver)              =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 10: SNDU Format for a Test SNDU

5.2 Bridge Frame SNDU Encapsulation

A bridged SNDU is of Type 1.  The payload includes a MAC source and Ether-Type field together with the contents of a bridged MAC frame. The SNDU has the format shown in figures 11 and 12.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0|        Length  (15b)        |         Type = 0x0001         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              Receiver Destination Address  (6B)               |

+                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               +

|                MAC Destination Address  (6B)                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    MAC Source Address  (6B)                   |

+                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |          EtherType (2B)       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

=                 (Contents of bridged MAC frame)               =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 11: SNDU Format for a Bridged Payload (D=0)

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1|        Length  (15b)        |         Type = 0x0001         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   MAC Destination Address  (6B)               |

+                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               +

|                     MAC Source Address  (6B)                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          EtherType (2B)       |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               +

|                                                               |

=                 (Contents of bridged MAC frame)               =

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             (CRC-32)                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 12: SNDU Format for a Bridged Payload (D=1)

Note: The final two bytes of the bridging header also carry a Type field (see section 5). In this special case, the extension mandatory header format permits this to carry a LLC Length field, specified by IEEE 802 [LLC] rather than an IANA assigned value.

When an NPA address is specified (D=0), Receivers MUST discard all SNDUs that carry an NPA address that does NOT match their own NPA address (or a broadcast/multicast address), the payload of the remaining SNDUs are processed by the bridging rules that follow. An SNDU without an NPA address (D=1) results in a Receiver performing bridging processing on the payload of all received SNDUs.

The MAC addresses in the frame being bridged SHOULD be assigned according to the rules specified by the IEEE and may denote unknown, unicast, broadcast, and multicast link addresses. These MAC addresses denote the intended recipient in the destination LAN, and therefore have a different function to the NPA addresses carried in the SNDU header. The EtherType field of a frame is defined according to Ethernet/LLC [LLC].

A frame Type < 1536 for a bridged frame, introduces a LLC Length field. The Receiver MUST check this length and discard any frame with a length greater than permitted by the SNDU payload size.

In normal operation, it is expected that any padding appended to the Ethernet frame SHOULD be removed prior to forwarding. This requires the sender to be aware of such Ethernet padding (e.g. LLC).

Ethernet frames received at the Encapsulator for onward transmission over ULE carry a Local Area Network Frame Check sequence, LAN FCS, field (e.g. CRC-32 for Ethernet). The Encapsulator MUST check the LAN-FCS value of all frames received, prior to further processing.  Frames received with an invalid LAN FCS MUST be discarded. After checking, the LAN FCS is then removed (i.e., it is NOT forwarded in the bridged SNDU).  As in other ULE frames, the Encapsulator appends a CRC-32 to the transmitted SNDU. At the Receiver, an appropriate LAN-FCS field will be appended to the bridged frame prior to onward transmission on the Ethernet interface.

This design is readily implemented using existing network interface cards, and does not introduce an efficiency cost by transmitting two integrity check fields for bridged frames. However, it also introduces the possibility that a frame corrupted within the processing performed at an Encapsulator and/or Receiver may not be detected by the final recipient(s) (i.e. such corruption would not normally result in an invalid LAN FCS).

6. Processing at the Encapsulator

The Encapsulator forms the PDUs queued for transmission into SNDUs by adding a header and trailer to each PDU (section 4). It then segments the SNDU into a series of TS Packet payloads (figure 9). These are transmitted using a single TS Logical Channel over a TS Multiplex. The TS Multiplex may be processed by a number of MPEG-2 (re)multiplexors before it is finally delivered to a Receiver.

+------+--------------------------------+------+

| ULE  |        Protocol Data Unit      | ULE  |

|Header|                                |CRC-32|

+------+--------------------------------+------+

/         /                              \       \

/         /                                \       \

/         /                                  \       \

+--------+---------+   +--------+---------+   +--------+---------+

|MPEG-2TS| MPEG-2  |...|MPEG-2TS| MPEG-2  |...|MPEG-2TS| MPEG-2  |

| Header | Payload |   | Header | Payload |   | Header | Payload |

+--------+---------+   +--------+---------+   +--------+---------+

Figure 13: Encapsulation of a SNDU into a series of TS Packets

6.1 SNDU Encapsulation

When an Encapsulator has not previously sent a TS Packet for a specific TS Logical Channel, or after an idle period, it starts to send a SNDU in the first available TS Packet.  This first TS Packet generated MUST carry a PUSI value of 1. It MUST also carry a Payload Pointer value of zero indicating the SNDU starts in the first available byte of the TS Packet payload.

The Encapsulation MUST ensure that all TS Packets set the MPEG-2 Continuity Counter carried in the TS Packet header, according to [ISO-MPEG].  This value MUST be incremented by one (modulo 16) for each successive fragment/complete SNDU sent using a TS Logical Channel.

An Encapsulator MAY decide not to immediately send another SNDU, even if space is available in a partially filled TS Packet. This procedure is known as Padding (figure 11). It informs the Receiver that there are no more SNDUs in this TS Packet payload. The End Indicator is followed by zero or more unused bytes until the end of the TS Packet payload. All unused bytes MUST be set to the value of 0xFF, following current practice in MPEG-2 [ISO-DSMCC]. The padding procedure trades decreased efficiency against improved latency.

+-/------------+

|  SubNetwork  |

|     DU 3     |

+-/------------+

\        \

\        \

\        \

+--------+--------+--------+----------+

|MPEG-2TS| End of | 0xFFFF |  Unused  |

| Header | SNDU 3 |        |  Bytes   |

+--------+--------+--------+----------+

PUSI=0            ULE

End

Indicator

Figure 14: A TS Packet carrying the end of SNDU 3, followed by an End Indicator.

Alternatively, when more packets are waiting at an Encapsulator, and a TS Packet has sufficient space remaining in the payload, the Encapsulator can follow a previously encapsulated SNDU with another SNDU using the next available byte of the TS Packet payload (see 6.2). This is called Packing (figure 15).

+-/----------------+       +----------------/-+

|   Subnetwork     |       |   Subnetwork     |

|      DU 1        |       |      DU 2        |

+-/----------------+       +----------------/-+

\        \     /          /\

\        \   /          /  \

\        \ /          /    \. . .

+--------+--------+--------+----------+

|MPEG-2TS| Payload| end of | start of |

| Header | Pointer| SNDU 1 | SNDU 2   |

+--------+--------+--------+----------+

PUSI=1     |              ^

|              |

+--------------+

Figure 15: A TS Packet with the end of SNDU 1, followed by SNDU 2.

6.2 Procedure for Padding and Packing

Five possible actions may occur when an Encapsulator has completed encapsulation of an SNDU:

(i) If the TS Packet has no remaining space, the Encapsulator transmits this TS Packet. It starts transmission of the next SNDU in a new TS Packet. (The standard rules [ISO-MPEG] require the header of this new TS Packet to carry a PUSI value of 1, and a Payload Pointer value of 0x00.)

(ii) If the TS Packet carrying the final part of a SNDU has one byte of unused payload, the Encapsulator MUST place the value 0xFF in this final byte, and transmit the TS Packet. This rule provides a simple mechanism to resolve the complex behaviour that may arise when the TS Packet has no PUSI set. To send another SNDU in the current TS Packet, would otherwise require the addition of a Payload Pointer that would consume the last remaining byte of TS Packet payload.  The behaviour follows similar practice for other MPEG-2 payload types [ISO-DSMCC]. The Encapsulator MUST start transmission of the next SNDU in a new TS Packet. (The standard rules require the header of this new TS Packet to carry a PUSI value of 1 and a Payload Pointer value of 0x00.)

(iii) If the TS Packet carrying the final part of a SNDU has exactly two bytes of unused payload, and the PUSI was NOT already set, the Encapsulator MUST place the value 0xFFFF in this final two bytes, providing an End Indicator (section 4.3), and transmit the TS Packet. This rule prevents fragmentation of the SNDU Length Field over two TS Packets. The Encapsulator MUST start transmission of the next SNDU in a new TS Packet. (The standard rules require the header of this new TS Packet to carry a PUSI value of 1 and a Payload Pointer value of 0x00.)

(iv) If the TS Packet has more than two bytes of unused payload, the Encapsulator MAY transmit this partially full TS Packet but MUST first place the value 0xFF in all remaining unused bytes (i.e. setting an End Indicator followed by Padding). The Encapsulator MUST start transmission of the next SNDU in a new TS Packet. (The standard rules [ISO-MPEG] require the header of this new TS Packet to carry a PUSI value of 1 and a Payload Pointer value of 0x00.)

(v) If at least two bytes are available for payload data in the TS Packet payload (i.e. three bytes if the PUSI was NOT previously set, and two bytes if it was previously set), the Encapsulator MAY encapsulate further queued PDUs, by starting the next SNDU in the next available byte of the current TS Packet payload. The PUSI MUST be set.  When the Encapsulator packs further SNDUs into a TS Packet where the PUSI has NOT already been set, this requires the PUSI to be updated (set to 1) and an 8-bit Payload Pointer MUST be inserted in the first byte directly following the TS Packet header. The value MUST be set to the position of the byte following the end of the first SNDU in the TS Packet payload. If no further PDUs are available, an Encapsulator MAY wait for additional PDUs to fill the incomplete TS Packet. The maximum period of time an Encapsulator can wait, known as the Packing Threshold, MUST be bounded and SHOULD be configurable in the Encapsulator. If sufficient additional PDUs are NOT received to complete the TS Packet within the Packing Threshold, the Encapsulator MUST insert an End Indicator (using rule iv).

Use of the Packing method (v) by an Encapsulator is optional, and may be determined on a per-session, per-packet, or per-SNDU basis.

When a SNDU is less than the size of a TS Packet payload, a TS Packet may be formed that carries a PUSI value of one and also an End Indicator (using rule iv).

7. Receiver Processing

A Receiver tunes to a specific TS Multiplex and sets a receive filter to accept all TS Packets with a specific PID.  These TS Packets are associated with a specific TS Logical Channel and are reassembled to form a stream of SNDUs.  A single Receiver may be able to receive multiple TS Logical Channels, possibly using a range of TS Multiplexes.  In each case, reassembly MUST be performed independently for each TS Logical Channel. To perform this reassembly, the Receiver may use a buffer to hold the partially assembled SNDU, referred to here as the Current SNDU buffer. Other implementations may choose to use other data structures, but MUST provide equivalent operations.

Receipt of a TS Packet with a PUSI value of 1 indicates that the TS Packet contains the start of a new SNDU.  It also indicates the presence of the Payload Pointer (indicating the number of bytes to the start of the first SNDU in the TS-Packet currently being reassembled). It is illegal to receive a Payload Pointer value greater than 181, and this MUST cause the SNDU reassembly to be aborted and the Receiver to enter the Idle State. This event SHOULD be recorded as a payload pointer error.

A Receiver MUST support the use of both the Packing and Padding method for any received SNDU, and MUST support reception of SNDUs with or without a Destination Address Field (i.e. D=0 and D=1).

7.1 Idle State

After initialisation, errors, or on receipt of an End Indicator, the Receiver enters the Idle State. In this state, the Receiver discards all TS Packets until it discovers the start of a new SNDU, when it then enters the Reassembly State. Figure 16 outlines these state transitions:

+-------+

| START |

+---+---+

|

\/

+----------+

\|   Idle   |/

+-------/|   State  |\-------+

Insufficient |        +----+-----+        |

unused space |             | PUSI set     | MPEG-2 TS Error

or           |            \/              | or

End Indicator|        +----------+        | SNDU Error

|        |Reassembly|        |

+--------|  State   |--------+

+----------+

Figure 16: Receiver state transitions

7.1.1 Idle State Payload Pointer Checking

A Receiver in the Idle State MUST check the PUSI value in the header of all received TS Packets. A PUSI value of 1 indicates the presence of a Payload Pointer. Following a loss of synchronisation, values between 0 and 181 are permitted, in which case the Receiver MUST discard the number of bytes indicated by the Payload Pointer from the start of the TS Packet payload, before leaving the Idle State. It then enters the Reassembly State, and starts reassembly of a new SNDU at this point.

7.2 Processing of a Received SNDU

When in the Reassembly State, the Receiver reads a 2 byte SNDU Length Field from the TS Packet payload. If the value is less than or equal to 4, or equal to 0xFFFF, the Receiver discards the Current SNDU and the remaining TS Packet payload and returns to the Idle State. Receipt of an invalid Length Field is an error event and SHOULD be recorded as an SNDU length error.

If the Length of the Current SNDU is greater than 4, the Receiver accepts bytes from the TS Packet payload to the Current SNDU buffer until either Length bytes in total are received, or the end of the TS Packet is reached (see also 7.2.1). When Current SNDU length equals the value of the Length Field, the Receiver MUST calculate and verify the CRC value (section 4.6). SNDUs that contain an invalid CRC value MUST be discarded. Mismatch of the CRC is an error event and SHOULD be recorded as a CRC error. The under-lying physical-layer processing (e.g. forward error correction coding) often results in patterns of errors, rather than since bit errors, so the Receiver needs to be robust to arbitrary patterns of corruption to the TS Packet and payload, including potential corruption of the PUSI, PP, and SNDU Length fields. Therefore, a Receiver SHOULD discard the remaining TS Packet payload (if any) following a CRC musmatch and return to the Idle State.

When the Destination Address is present (D=0), the Receiver accepts SNDUs that match one of a set of addresses specified by the Receiver (this includes the NPA address of the Receiver, the NPA broadcast address and any required multicast NPA addresses). The Receiver MUST silently discard an SNDU with an unmatched address.

After receiving a valid SNDU, the Receiver MUST check the Type Field (and process any Type 1 Extension Headers). The SNDU payload is then passed to the next protocol layer specified. An SNDU with an unknown Type value < 1536 MUST be discarded. This error event SHOULD be recorded as a SNDU type error.

The Receiver then starts reassembly of the next SNDU. This MAY directly follow the previously reassembled SNDU within the TS Packet payload.

(i) If the Current SNDU finishes at the end of a TS Packet payload, the Receiver MUST enter the Idle State.

(ii) If only one byte remains unprocessed in the TS Packet payload after completion of the Current SNDU, the Receiver MUST discard this final byte of TS Packet payload. It then enters the Idle State. It MUST NOT record an error when the value of the remaining byte is identical to 0xFF.

(iii) If two or more bytes of TS Packet payload data remain after completion of the Current SNDU, the Receiver accepts the next 2 bytes and examines if this is an End Indicator. When an End Indicator is received, a Receiver MUST silently discard the remainder of the TS Packet payload and transition to the Idle State. Otherwise this is the start of the next Packed SNDU, and the Receiver continues by processing this SNDU.

7.2.1 Reassembly Payload Pointer Checking

A Receiver that has partially received a SNDU (in the Current SNDU buffer) MUST check the PUSI value in the header of all subsequent TS Packets with the same PID (i.e. same TS Logical Channel). If it receives a TS Packet with a PUSI value of 1, it MUST then verify the Payload Pointer. If the Payload Pointer does NOT equal the number of bytes remaining to complete the Current SNDU, i.e., the difference between the SNDU Length field and the number of reassembled bytes, the Receiver has detected a delimiting error.

Following a delimiting error, the Receiver MUST discard the partially assembled SNDU (in the Current SNDU buffer), and SHOULD record a reassembly error. It MUST then re-enter the Idle State.

7.3 Other Error Conditions

The Receiver SHOULD check the MPEG-2 Transport Error Indicator carried in the TS Packet header [ISO-MPEG].  This flag indicates a transmission error for a TS Logical Channel. If the flag is set to a value of one, a transmission error event SHOULD be recorded. Any partially received SNDU MUST be discarded. The Receiver then enters the Idle State.

The Receiver MUST check the MPEG-2 Continuity Counter carried in the TS Packet header [ISO-MPEG]. If two (or more) successive TS Packets within the same TS Logical Channel carry the same Continuity Counter value, the duplicate TS Packets MUST be silently discarded. If the received value is NOT identical to that in the previous TS Packet, and it does NOT increment by one for successive TS Packets (modulo 16), the Receiver has detected a continuity error. Any partially received SNDU MUST be discarded. A continuity counter error event SHOULD be recorded. The Receiver then enters the Idle State.

Note that an MPEG2-2 Transmission network is permitted to carry

duplicate TS Packets [ISO-MPEG], which are normally detected by the

MPEG-2 Continuity Counter.  A Receiver that does not perform the above Continuity Counter check, would accept duplicate copies of TS Packets to the reassembly procedure. In most cases, the SNDU CRC-32 integrity check will result in discard of these SNDUs, leading to unexpected PDU loss, however in some cases, duplicate PDUs (fitting into one TS Packet) could pass undetected to the next layer protocol.

8. Summary

This document defines an Ultra Lightweight Encapsulation (ULE) to perform efficient and flexible support for IPv4 and IPv6 network services over networks built upon the MPEG-2 Transport Stream (TS). The encapsulation is also suited to transport of other protocol packets and bridged Ethernet frames.

ULE also provides an Extension Header format and defines an associated IANA registry for efficient and flexible support of both mandatory and optional SNDU headers. This allows for future extension of the protocol, while providing backwards capability with existing implementations. In particular, Optional Extension Headers may safely be ignored by drivers that do not implement them, or choose not to process them.

9. Acknowledgments

This draft is based on a previous draft authored by: Horst D. Clausen, Bernhard Collini-Nocker, Hilmar Linder, and Gorry Fairhurst. The authors wish to thank the members of the ip-dvb mailing list for their input provided. In particular, the many comments received from Patrick Cipiere, Wolgang Fritsche, Hilmar Linder, Alain Ritoux, and William Stanislaus. Alain also provided the original examples of usage.

- 10. Security Considerations

The security considerations for ULE resemble those that arise when the exiting Multi-Protocol Encapsulation (MPE) is used.  ULE does not add specific new threats that will impact the security of the general Internet.

There is a known security issue with un-initialised stuffing bytes.  In ULE, these bytes are set to 0xFF (normal practice in MPEG-2).

There are known integrity issues with the removal of the LAN FCS in a bridged networking environment. The removal for bridged frames exposes the traffic to potentially undetected corruption while being processed by the Encapsulator and/or Receiver.

There is a potential security issue when a Receiver receives a PDU with two length fields:  The Receiver would need to validate the actual length and the Length Field and ensure that inconsistent values are not propagated by the network. In ULE, this is avoided by including only one SNDU Length Field.  However, this issue still arises in bridged LLC frames, and frames with a LLC Length greater than the SNDU payload size MUST be discarded, and a SNDU payload length error SHOULD be recorded.

ULE supports optional link level encryption of the SNDU payload.  This is as an additional security mechanism to IP, transport or application layer security - not a replacement [ID-ipdvb-arch]. The approach is generic and decouples the encapsulation from future security extensions. The operation provides functions that resemble those currently used with the MPE encapsulation.

A ULE Mandatory Extension Header may in future be used to define a method to perform link encryption. Additional security control fields may be provided as a part of the Extension Header, e.g. to associate an SNDU with one of a set of Security Association (SA) parameters. As a part of the encryption process, it may also be desirable to authenticate some/all of the SNDU headers. The method of encryption and the way in which keys are exchanged is beyond the scope of this specification, as also are the definition of the SA format and that of the related encryption keys.

- 11. References

11.1 Normative References

[ISO-MPEG] ISO/IEC DIS 13818-1 "Information technology -- Generic coding of moving pictures and associated audio information: Systems", International Standards Organisation (ISO).

[RFC2026] Bradner, S., "The Internet Standards Process - Revision 3", BCP 9, RFC 2026, BCP 9, 1996.

[RFC2119] Bradner, S., "Key Words for Use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, 1997.

[RFC3667] Bradner, S., "IETF Rights in Contributions", BCP 78, RFC 3667, February 2004.

[RFC3668] Bradner, S., "Intellectual Property Rights in IETF Technology", BCP 79, RFC 3668, February 2004.

11.2 Informative References

[ID-ipdvb-arch] "Requirements for transmission of IP datagrams over MPEG-2 networks", Internet Draft, Work in Progress.

[ATSC] A/53, "ATSC Digital Television Standard", Advanced Television Systems Committee (ATSC), Doc. A/53, 1995.

[ATSC-DAT] A/90, "ATSC Data Broadcast Standard", Advanced Television Systems Committee (ATSC), Doc. A/090, 2000.

[ATSC-DATG] A/91, "Recommended Practice: Implementation Guidelines for the ATSC Data Broadcast Standard", Advanced Television Systems Committee (ATSC), Doc. A/91, 2001.

[ATSC-G] A/54, "Guide to the use of the ATSC Digital Television Standard", Advanced Television Systems Committee (ATSC), Doc. A/54, 1995.

[ATSC-PSIP-TC] A/65A, "Program and System Information Protocol for Terrestrial Broadcast and Cable", Advanced Television Systems Committee (ATSC), Doc. A/65A, 23 Dec 1997, Rev. A, 2000.

[ATSC-S] A/80, "Modulation and Coding Requirements for Digital TV (DTV) Applications over Satellite", Advanced Television Systems Committee (ATSC), Doc. A/80, 1999.

[CLC99] Clausen, H., Linder, H., and Collini-Nocker, B., "Internet over Broadcast Satellites", IEEE Commun. Mag. 1999, pp.146-151.

[ETSI-DAT] EN 301 192 "Specifications for Data Broadcasting", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBC] EN 300 800 "Digital Video Broadcasting (DVB); DVB interaction channel for Cable TV distribution systems (CATV)", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS] EN 301 421 "Digital Video Broadcasting (DVB); Modulation and Coding for DBS satellite systems at 11/12 GHz", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBT] EN 300 744 "Digital Video Broadcasting (DVB); Framing structure, channel coding and modulation for digital terrestrial television (DVB-T)", European Telecommunications Standards Institute (ETSI).

[ETSI-RCS] ETSI 301 791 "Digital Video Broadcasting (DVB); Interaction Channel for Satellite Distribution Systems", European Telecommunications Standards Institute (ETSI).

[ISO-DSMCC] ISO/IEC IS 13818-6 "Information technology -- Generic coding of moving pictures and associated audio information -- Part 6: Extensions for DSM-CC", International Standards Organisation (ISO).

[ITU-I363] ITU-T I.363.5 B-ISDN ATM Adaptation Layer Specification Type AAL5, International Standards Organisation (ISO), 1996.

[LLC] "IEEE Logical Link Control" (ANSI/IEEE Std 802.2/ ISO 8802.2), 1985.

[RFC3077] E. Duros, W. Dabbous, H. Izumiyama, Y. Zhang, "A Link Layer Tunneling Mechanism for Unidirectional Links", RFC3077, Proposed Standard, 2001.

[RFC3309] Stone, J., R. Stewart, D. Otis. "Stream Control Transmission Protocol (SCTP) Checksum Change". RFC3095, Proposed Standard, 2001.

12. Authors' Addresses

Godred Fairhurst

Department of Engineering

University of Aberdeen

Aberdeen, AB24 3UE

UK

Email: gorry@erg.abdn.ac.uk

Web: http://www.erg.abdn.ac.uk/users/Gorry

Bernhard Collini-Nocker

Department of Scientific Computing

University of Salzburg

Jakob Haringer Str. 2

5020 Salzburg

Austria

Email: bnocker@cosy.sbg.ac.at

Web: http://www.scicomp.sbg.ac.at/

13. IPR Notices

13.1 Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any

Intellectual Property Rights or other rights that might be claimed

to pertain to the implementation or use of the technology described

in this document or the extent to which any license under such

rights might or might not be available; nor does it represent that

it has made any independent effort to identify any such rights.

Information on the procedures with respect to rights in RFC

documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any assurances of licenses to be made available, or the result of an

attempt made to obtain a general license or permission for the use

of such proprietary rights by implementers or users of this

specification can be obtained from the IETF on-line IPR repository

at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights that may cover technology that may be required to implement

this standard.  Please address the information to the IETF at ietf-

ipr@ietf.org.

13.2 Disclaimer of Validity

This document and the information contained herein are provided on an "AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY AND THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

14. Copyright Statement

Copyright (C) The Internet Society (2004).  This document is

subject to the rights, licenses and restrictions contained in

BCP 78, and except as set forth therein, the authors retain all

their rights.

15. IANA Considerations

This document will require IANA involvement.

The ULE Next-Header type field defined in this document requires a registry:

ULE Next-Header registry

This registry allocates values 0-512 (decimal).

15.1 IANA Guidelines

The following contains the IANA guidelines for management of the ULE Next-Header registry. This registry allocates values decimal 0-512 (0x0000-0x01FF, hexadecimal). It MUST NOT allocate values greater than 0x01FF (decimal).

It subdivides the Next-Header registry in the following way:

- 0-255 (decimal) IANA assigned values indicating Mandatory Extension Headers (or link-dependent type fields) for ULE, requiring expert review leading to prior issue of an IETF RFC. This specification must define the value, and the name associated with the Extension Header. It must also define the need for the extension and the intended use. The size of the Extension Header must also be specified.

Assignments made in this document:

Type      Name                             Reference

0:       Test-SNDU                        Section 4.7.4.

1:       Bridged-SNDU                     Section 4.7.5.

- 256-511 (decimal) IANA assigned values indicating Optional Extension Headers for ULE, requiring expert review leading to prior issue of an IETF RFC. This specification must define the value, and the name associated with the Extension Header. The entry must specify range of allowable H-LEN values that are permitted (in the range 1-5). It must also define the need for the extension and the intended use.

Assignments made in this document:

Type      Name                    H-LEN    Reference

256:      Extension-Padding       1-5     Section 5.

ANNEXE A: Informative Appendix

This appendix provides some examples of use. The appendix is informative. It does not provide a description of the protocol.  The examples provide the complete TS Packet sequence for some sample encapsulated IP packets.

The specification of the TS Packet header operation and field values is provided in [ISO-MPEG].  The specification of ULE is provided in the body of this document.

The key below is provided for the following examples.

HDR    4B TS Packet Header

PUSI   Payload Unit Start Indicator

PP     Payload Pointer

***    TS Packet Payload Pointer (PP)

Example A.1: Two 186B PDUs.

SNDU A is 200 bytes (including destination MAC address)

SNDU B is 200 bytes (including destination MAC address)

The sequence comprises 3 TS Packets:

SNDU

PP=0      Length

+-----+------+------+------+-   -+------+

| HDR | 0x00 | 0x00 | 0xC4 | ... | A182 |

+-----+----*-+-*----+------+-   -+------+

PUSI=1     *   *

*****

SNDU

PP=17           CRC for A     Length

+-----+------+------+-   -+--- --+------+------+-   -+------+

| HDR | 0x11 | A183 | ... | A199 | 0x00 | 0xC4 | ... | B165 |

+-----+----*-+------+-   -+------+-*----+------+-   -+------+

PUSI=1     *                       *

*************************

End     Stuffing

CRC for A Indicator   Bytes

+-----+------+-   -+------+----+----+-   -+----+

| HDR | B166 | ... | B199 |0xFF|0xFF| ... |0xFF|

+-----+------+-   -+------+----+----+-   -+----+

PUSI=0

Example A.2: Usage of last byte in a TS-Packet

SNDU A is 183 bytes

SNDU B is 182 bytes

SNDU C is 181 bytes

SNDU D is 185 bytes

The sequence comprises 4 TS Packets:

SNDU

PP=0      Length     CRC for A

+-----+------+------+------+-   -+------+

| HDR | 0x00 | 0x00 | 0x63 | ... | A182 |

+-----+----*-+-*----+------+-   -+------+

PUSI=1     *   *

*****

SNDU                  Unused

PP=0      Length       CRC for B  byte

+-----+------+------+------+-   -+------+------+

| HDR | 0x00 | 0x00 | 0x62 | ... | B181 | 0xFF |

+-----+---*--+-*----+------+-   -+------+------+

PUSI=1    *    *

******

SNDU                       SNDU

PP=0      Length      CRC for C      Length

+-----+------+------+------+-   -+------+------+------+

| HDR | 0x00 | 0x00 | 0x61 | ... | C180 | 0x00 | 0x65 |

+-----+---*--+-*----+------+-   -+------+------+------+

PUSI=1    *    *

******           Unused

byte

+-----+------+-   -+------+------+

| HDR | D002 | ... | D184 | 0xFF |

+-----+------+-   -+------+------+

PUSI=0

Example A.3: Large SNDUs

SNDU A is 732 bytes

SNDU B is 284 bytes

The sequence comprises 6 TS Packets:

SNDU

PP=0      Length

+-----+------+------+------+-   -+------+

| HDR | 0x00 | 0x02 | 0xD8 | ... | A182 |

+-----+---*--+-*----+------+-   -+------+

PUSI=1    *    *

******

+-----+------+-   -+------+

| HDR | A183 | ... | A366 |

+-----+------+-   -+------+

PUSI=0

+-----+------+-   -+------+

| HDR | A367 | ... | A550 |

+-----+------+-   -+------+

PUSI=0

SNDU

PP=181         CRC for A      Length

+-----+------+------+-   -+------+------+------+

| HDR | 0xB5 | A551 | ... | A731 | 0x01 | 0x18 |

+-----+---*--+------+-   -+------+*-----+------+

PUSI=1    *                       *

*************************

+-----+------+-   -+------+

| HDR | B002 | ... | B185 |

+-----+------+-   -+------+

PUSI=0

End          Stuffing

Indicator        Bytes

+-----+------+-   -+------+------+------+-   -+------+

| HDR | B186 | ... | B283 | 0xFF | 0xFF | ... | 0xFF |

+-----+------+-   -+------+------+------+-   -+------+

PUSI=0

Example A.4: Packing of SNDUs

SNDU A is 200 bytes

SNDU B is 60 bytes

SNDU C is 60 bytes

The sequence comprises two TS Packets:

SNDU

PP=0      Length

+-----+------+------+------+-   -+------+

| HDR | 0x00 | 0x00 | 0xC4 | ... | A182 |

+-----+----*-+-*----+------+-   -+------+

PUSI=1     *   *  +      +

*****  ++++++++

+

+++++++++++++++++

+   SNDU

PP=17           CRC for A  +  Length

+-----+------+------+-   -+------+-+----+------+-

| HDR | 0x11 | A183 | ... | A199 | 0x00 | 0x38 | ...

+-----+----*-+------+-   -+------+*-----+------+-

PUSI=1     *                      *  +       +

************************  +++++++++

+

+++++++++++++++++++++++++++++++++++++++

+

+                  SNDU                       End      Stuffing

+                 Length                   Indicator     bytes

+    -+------+------+------+  -+------+------+------+- -+------+

+ ... | B59  | 0x00 | 0x38 |...| C59  | 0xFF | 0xFF |...| 0xFF |

+    -+------+-+----+------+  -+------+-+----+------+- -+------+

+              +  +      +              +

+              +  ++++++++              +

+              +   +                    +

++++++++++++++++   ++++++++++++++++++++++

*** TS Packet Payload Pointer (PP)

+++ ULE Length Indicator

Example A.5: Three 44B PDUs.

SNDU A is 52 bytes (no destination MAC address)

SNDU B is 52 bytes (no destination MAC address)

SNDU C is 52 bytes (no destination MAC address)

The sequence comprises 1 TS Packet:

SNDU

PP=0      Length

+-----+------+------+------+-   -+-----+------+-----+-   -+-----+-

| HDR | 0x00 | 0x80 | 0x34 | ... | A51 |0x80 | 0x34 | ... | B51 | ..

+-----+----*-+-*----+------+-   -+-----+-*----+-----+-   -+-----+-

PUSI=1     *   *

*****

End        Stuffing

Indicator     bytes

-----+------+-   -+-----+---------+- -+------+

...	0x80 | 0x34 | ... | C51 |0xFF|0xFF|   | 0xFF |

-*---+------+-   -+-----+---------+- -+------+

ANNEXE B: Informative Appendix - SNDU Encapsulation

An example of ULE encapsulation carrying an ICMPv6 packet generated by ping6.

ULE SNDU Length  : 		 63 decimal

D-bit value	:                0 (NPA Present)

ULE Protocol Type : 		 0x86dd (IPv6)

Destination ULE NPA Address:  00:01:02:03:04:05

ULE CRC32 : 			 0x4709a744

Source IPv6:                  2001:660:3008:1789::5

Destination IPv6:             2001:660:3008:1789::6

SNDU contents (including CRC-32):

0000: 00 3f 86 dd 00 01 02 03 04 05 60 00 00 00 00 0d

0016: 3a 40 20 01 06 60 30 08 17 89 00 00 00 00 00 00

0032: 00 05 20 01 06 60 30 08 17 89 00 00 00 00 00 00

0048: 00 06 80 00 9d 8c 06 38 00 04 00 00 00 00 00 47

0064: 09 a7 44