Internet Engineering Task Force		        Marie J. Montpetit ed.

Internet Draft                                  MJMontpetit.com, USA

Document: draft-ipdvb-arch-00.txt                    Gorry Fairhurst

University of Aberdeen, U.K.

Horst D. Clausen

Bernhard Collini-Nocker

Hilmar Linder

University of Salzburg, Austria

Category: Informational                                    July 2004

A Framework for transmission of IP datagrams over MPEG-2 Networks

Status of this Memo

By submitting this Internet-Draft, we certify that any applicable

patent or other IPR claims of which we are aware have been disclosed,

or will be disclosed, and any of which we become aware will be

disclosed, in accordance with RFC 3668.

By submitting this Internet-Draft, we accept the provisions of

Section 3 of RFC 3667 (BCP 78).

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups. Note that

other groups may also distribute working documents as Internet-

Drafts. Internet-Drafts are draft documents valid for a maximum of

six months and may be updated, replaced, or obsoleted by other

documents at any time. It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress." The list of current Internet-Drafts can be accessed at

http://www.ietf.org/ietf/1id-abstracts.txt. The list of Internet-

Draft Shadow Directories can be accessed at

http://www.ietf.org/shadow.html.

Copyright (C) The Internet Society (2004), All Rights Reserved

Abstract

This document describes a new architecture for the transport of IP

Datagrams over ISO MPEG-2 Transport Streams (TS). The MPEG-2 TS has

been widely accepted not only for providing digital TV services,

but also as a subnetwork technology for building IP networks.

Examples of systems using MPEG-2 include the Digital Video

Broadcast (DVB)and Advanced Television Systems Committee (ATSC)

Standards for Digital Television.

The document identifies the need for a set of Internet standards

defining the interface between the MPEG-2 Transport Stream and an

IP subnetwork. It suggests a new encapsulation method for IP

datagrams and proposes protocols to perform IPv6/IPv4 address

resolution, to associate IP packets with the properties of the

Logical Channels provided by an MPEG-2 TS.

Expires November 2004                                       [page 1]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

Table of Contents

1. Introduction

2. Conventions Used in this Document

3. Architecture

4. Encapsulation Protocol

5. Address Resolution Functions

6. Multicast Support

7. Summary

8. Security Considerations

9. Acknowledgments

10. References

11. Author's Addresses

12. IANA Considerations

Appendices

Expires November 2004                                       [page 2]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

Document History

-00 Current version

Expires November 2004                                      [page 3]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

1. Introduction

This document identifies requirements and an architecture for

transport of IP Datagrams over ISO MPEG-2 Transport Streams [MPEG2].

The prime focus is the efficient and flexible delivery of IP

services over those subnetworks that use the MPEG-2 transport

stream.

Hence, the architecture is designed to be compatible with services

based on MPEG-2, for example the Digital Video Broadcast (DVB)

architecture, the Advanced Television Systems Committee (ATSC)

system [ATSC; ATSC-G], and other similar MPEG-2 based transmission

systems. Such systems typically provide unidirectional (simplex)

physical and link layer standards, and have been defined for a wide

range of physical media (e.g. Terrestrial TV [ETSI-DVBT; ATSC-PSIP-

TC], Satellite TV [ETSI-DVBS; ATSC-S] and Cable Transmission [ETSI-

DVBC; ATSC-PSIP-TC]).

+---------+-+-+-+-+------+--------+---+--+--+

|         |T|V|A|O|  O   |        | O |S |O |

|         |e|i|u|t|  t   |        | t |I |t |

|         |l|d|d|h|  h   |   IP   | h |  |h |

|Protocols|e|e|i|e|  e   |        | e |T |e |

|native   |t|o|o|r|  r   |        | r |a |r |

|over     |e| | | |      |        |   |b |  |

|MPEG-2 TS|x| | | |      |   +----+-+ |l |  |

|         |t| | | |      |   | MPE  | |e |  |

|         | | | | |   +--+---+------+ |  |  |

|         | | | | |   | AAL5 |Priv. | |  |  |

|         +-+-+-+-+---+------+      +-+--+--+

|         | PES   |  ATM     |Sect. |Section|

+---------+-------+----------+------+-------+

|               MPEG-2 TS                   |

+---------+-------+-------+-----------------+

|Satellite| Cable | D-TV  |    Other PHY    |

+---------+-------+-------+-----------------+

Figure 1: Overview of the MPEG-2 protocol stack

Although many MPEG-2 systems carry a mixture data types, MPEG-2

components may, and are, also used to build IP-only networks.

Often, those networks do not implement all parts of a DVB / ATSC

system, and may for instance support minimal, or no, signalling of

Service Information (SI) tables. Standard system components offer

advantages of improved interoperability and larger deployment.

1.1 Salient features of the Architecture

The architecture proposed in this document describes a set of

protocols to support transmission of IP packets over the MPEG-2 TS.

Key characteristics of these networks are that they may provide

link-level broadcast capability, and that many supported

applications require access to a very large number of subnetwork

nodes.

Expires November 2004                                       [page 4]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

Some or all of these protocols may also be applicable to other

subnetworks,e.g., other MPEG-2 transmission networks, regenerative

satellite links [ETSI-BSM], and some types of broadcast wireless

links. The key goals of the architecture are to reduce complexity

when using the system, while improving performance, increasing

flexibility for IP services, and providing opportunities for better

integration with IP services.

Since a majority of MPEG-2 transmission networks are bandwidth-

limited, encapsulation protocols must therefore add minimal

overhead to ensure good link efficiency while providing adequate

network services. They also needs to be simple to minimize

processing, robust to errors and security threats, and extensible

to a wide range of services.

In MPEG-2 systems, logical channels provide multiplexing,

addressing, and error reporting. The logical channel may also be

used to provide Quality of Service (QoS). Mapping functions are

required to relate Logical Channels to IP addresses, to map Logical

Channels to IP-level QoS, and to associate IP flows with specific

subnetwork capabilities.  An important feature of the proposed

architecture will be to provide these functions in a dynamic way,

allowing transparent integration with other IP-layer protocols.

Collectively, these will form a MPEG-2 TS address resolution

protocol suite.

Expires November 2004                                       [page 5]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

2. Conventions Used In This Document

Adaptation Field: Option control overhead or padding associated

with an MPEG-2 TS Packet. Examples of overhead are: TS Logical

Channel encryption details and clock (PCR) information to

synchronise a set of TS Logical Channels.

ATSC: Advanced Television Systems Committee [ATSC]. A set of

framework and associated standards for the transmission of video,

audio, and data, using the ISO MPEG-2 standard.

DSM-CC: Digital Storage Media Command and Control [ISO-DSMCC].

A formatting defined by the ISO MPEG-2 standard, which is carried

In an MPEG-2 private section.

DVB: Digital Video Broadcast [ETSI-DVB]. A set of framework and

associated standards for the transmission of video, audio, and

data, using the ISO MPEG-2 standard.

ENCAPSULATOR: A network device that receives Ethernet frames or

IP packets, and formats these for output as a transport stream

of TS Packets.

FORWARD DIRECTION: The dominant direction of data transfer over a

network path. Data transfer in the forward direction is called

"forward transfer".  Packets travelling in the forward direction

follow the forward path through the IP network.

MAC: Medium Access and Control of the Ethernet IEEE 802 standard

of protocols (see also NPA).

MPE: Multiprotocol Encapsulation [ETSI-DAT]. A scheme that

encapsulates Ethernet frames or IP Packets, creating a

DSM-CC Section. The Section will be sent in a series of TS

Packets over a TS Logical Channel.

MPEG-2: A set of standards specified by the Motion Picture

Experts Group (MPEG), and standardized by the International

Standards Organisation (ISO) [ISO-MPEG].

NPA: Network Point of Attachment. Addresses primarily used for

station (Receiver) identification within a local network (e.g.

IEEE MAC address).

PES: Packetized Elementary Stream. A format of MPEG-2 TS packet

payload usually used for video or audio information in MPEG-2

[ISO-MPEG].

Expires November 2004                                      [page 6]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

PID: Packet Identifier. A 13-bit field carried in the header of

all MPEG-2 Transport Stream packets [ISO-MPEG]. This is used to

identify the TS Logical Channel to which it belongs. A PID of 8191

(decimal)indicates a null packet that must be discarded by a

Receiver.

PRIVATE SECTION: A syntactic structure used for mapping all

service information (e.g. an SI table) into TS Packets.  A table

may be divided into a number of sections.  All sections of a table

must be carried over a single TS Logical Channel.

SI TABLE: Service Information Table. In this document, the term is

used to describe any table used to convey information about the

service carried in a TS Multiplex (e.g. [ISO-MPEG]). SI tables are

carried in MPEG-2 private sections.

STB: Set Top Box. A consumer equipment (Receiver) for reception of

digital TV services.

TS: Transport Stream [ISO-MPEG], a method of transmission at the

MPEG-2 level using TS Packets; it represents level 2 of the ISO/OSI

reference model. See also TS Logical Channel and TS Multiplex.

TS LOGICAL CHANNEL: A channel identified at the MPEG-2 level; it

represents level 2 of the ISO/OSI reference model. All packets sent

over a channel carry the same PID value.

TS MULTIPLEX: A set of MPEG-2 TS Logical Channels sent over a single

common physical bearer (i.e. a link transmitting at a specified

symbol rate, FEC setting, and transmission frequency).

TS PACKET: A fixed-length 188B unit of data sent over an MPEG-2

multiplex [ISO-MPEG]; it corresponds to the cells, of e.g. ATM

networks, and is frequently also referred to as a TS_cell.  Each TS

Packet carries a 4B header, plus optional overhead including a

pointer to the next payload header (PUSI), and an adaptation field.

Each TS packet carries a PID value to associate it with a single TS

Logical Channel.

Expires November 2004                                       [page 7]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

3. Architecture

The following sections introduce the components of the MPEG-2

Transmission network and relate these to a networking framework.

3.1 MPEG-2 Transmission Networks

There are many possible topologies for the MPEG-2 Transmission

Networks. A number of example scenarios are briefly described

below, and the following text relates specific functions to this

set of scenarios.

A) Broadcast TV and Radio Delivery

The principal service in the Broadcast TV and Radio Delivery

scenario is Digital TV and/or Radio and their associated data [ID-

MMUSIC-IMG]. Such networks typically contain two components: the

contribution feed and the broadcast part.  Contribution feeds

provide communication from a typically small number of individual

sites (usually at high quality) to the Hub of a broadcast network.

The traffic carried on contribution feeds is typically encrypted,

and is usually processed prior to being resent on the Broadcast part

of the network. The Broadcast part uses a star topology centred on

the Hub to reach a typically large numbers of down-stream Receivers.

Although such networks may provide IP transmission, they do not

necessarily provide access to the public Internet.

B) Broadcast Networks used as an ISP

Another scenario resembles that above, but includes the provision of

IP-services providing access to the public Internet. The IP traffic

in this scenario is typically not related to the digital TV/Radio

content, and the service may be operated by an independent operator

such as uni-directional File Delivery or bi-directional ISP access.

The IP service must adhere to the full system specification used

for the broadcast transmission, including allocation of PIDs, and

generation of appropriate MPEG-2 control information (e.g. DVB and

ATSC SI tables).

C) Uni-directional Star IP Scenario

The Uni-directional Star IP Scenario utilises a Hub station to

provide a data network delivering a common bit stream to medium-

sized groups of Receivers. MPEG-2 transmission technology provides

the forward physical and link layers for this transmission, the

return link (if required) is provided by other means. IP services

typically form the main proportion of the transmission traffic.

Such networks may not necessarily implement the MPEG-2 control

plane.

D) Data-cast overlay

The Data-cast overlay scenario employs MPEG-2 physical and link

layers to provide additional connectivity such as uni-directional

multicast to supplement an existing IP-based Internet service.

Examples of such a network includes IP Datacast to mobile wireless

receivers [ID-MMUSIC-IMG].

Expires November 2004                                       [page 8]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

E) Point-to-point links

Point-to-point connectivity may be provided using a pair of

transmit and receive interfaces supporting the MPEG-2 physical and

link layers.  Typically the transmission from a sender is received

by only one or a small number of Receive terminals. Examples

include the use of transmit/receive DVB-S terminals to provide

satellite links between ISPs utilising BGP routing.

F) Two-Way IP Networks

Two-Way IP networks are typically satellite-based and star-based

utilising a Hub station to deliver a common bit stream to medium-

sized groups of receivers. A bi-directional service is provided over

a common air-interface. The transmission technology in the forward

physical and link layers for this transmission is MPEG-2, which may

also be used in the return direction. Such systems also usually

include a control plane element to manage the (shared) return link

capacity. A concrete example is the DVB-RCS system. IP services

typically form the main proportion of the transmission traffic.

Scenarios A-D employ uni-directional MPEG-2 Transmission networks.

For satellite-based networks, these typically have a star topology,

with a central Hub providing service to large numbers of down-stream

Receivers. Terrestrial networks may employ several transmission Hubs

each serving a particular coverage cell with a community of

Receivers.

From an IP viewpoint, the service is typically either uni-

directional multicast, or a bi-directional service in which some

complementary link technology (e.g. Modem, LMDS, GPRS, ...) is used

to provide the return path from Receivers to the Internet.  Routing

in this case could be provided using Uni-Directional Link Routing

(UDLR) [RFC3077].

Note that only Scenarios A-B actually carry MPEG-2 video and audio

intended for reception by digital Set Top Boxes (STBs) as the

primary traffic. The other scenarios are IP-based data networks and

need not necessarily implement the MPEG-2 control plane.

Scenarios E-F provide two-way connectivity using MPEG-2

transmission.  Such networks provide direct support for bi-

directional protocols above and below the IP layer.

The complete MPEG-2 transmission network may be managed by a

transmission service operator. In such cases, the assignment of

addresses and TS Logical Channels at Receivers are usually under

the control of the service operator.  Examples include a TV

operator (Scenario A), or an ISP (Scenarios B-F).  MPEG-2

transmission networks are also used for private networks. These

typically involve a smaller number of Receivers and do not require

the same level of centralised control. Examples include companies

wishing to connect DVB-capable routers to form links within the

Internet (Scenario B).

Expires November 2004                                       [page 9]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

3.2 TS Logical Channels

An MPEG-2 transport multiplex offers a number of parallel channels,

which are known here as TS Logical Channels.  Each MPEG-2 TS Logical

Channel is uniquely identified by the Packet ID, PID, value carried

in the header of each MPEG-2 TS Packet. The PID value is a 13 bit

field and, thus, the number of available channels is limited to 8192

(decimal), some of which are reserved for transmission of SI tables.

Non-reserved TS Logical Channels may be use to carry audio[ISO-AUD],

video [ISO-VID], IP packets [ISO-DSMCC; ETSI-DAT; ATSC-DAT],

or other data [ISO-DSMCC; ETSI-DAT; ATSC-DAT]. The value 8191

(decimal) indicates a null packet, used to maintain the physical

bearer bit rate when there are no other MPEG-2 TS Packets to be

sent.

TS-LC-A-1         /---\--------------------/---\

\        /     \                  /     \

\      |       |                |       |

TS-LC-A-2    -----------   |                | -------------

--------------------   |                | -------------

|       |                |       |

/--------   /                 | -------------

/      \----/-------------------\----/

TS-LC-A-3/               MPEG-2 TS MUX A

/

TS-LC        /

------------X

\ TS-LC-B-3 /---\------------------------/---\

\         /     \                      /     \

\       |       |                    |       |

TS-LC-B-2    \-----------   |                    | ---------

--------------------   |                    | ---------

|       |                    |       |

/--------   /                     | ---------

/      \----/-----------------------\----/

/                 MPEG-2 TS MUX B

TS-LC-B-1

Figure 2: Example showing MPEG-2 TS Logical Channels carried over

2 MPEG-2 TS Multiplexes.

TS Logical Channels are independently numbered on each MPEG-2 TS

Multiplex (MUX).  In most cases the data sent over the TS Logical

Channels will differ for different multiplexes. The above figure

shows a set of TS Logical Channels sent using two MPEG-2 TS

Multiplexes (A and B).

There are cases where the same data may be distributed over two or

more multiplexes (e.g., some SI tables; multicast content which

needs to be received by Receivers tuned to either MPEG-2 TS; unicast

data were the Receiver may be in either/both two potentially

overlapping MPEG-2 transmission cells). In figure 2, each multiplex

Expires November 2004                                      [page 10]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

carries 3 MPEG-2 TS Logical Channels. These Logical Channels may

differ (TS-LC-A-1, TS-LC-A-2, TS-LC-B-2, TS-LC-B-1), or may be

common to both MPEG-2 TS Multiplexes (i.e. TS-LC-A-3 and TS-LC-B-3

carry identical content).

As can been seen above, there are similarities between the way PIDs

are used and the operation of virtual channels in ATM. However,

unlike ATM, a PID defines a unidirectional broadcast channel and not

a point-to-point link. Contrary to ATM, there is, as yet, no

specified standard interface for MPEG-2 connection setup, or for

signalling mappings of IP flows to PIDs, or to set the Quality of

Service, QoS, assigned to an TS Logical Channel.

3.3  Multiplexing and Re-Multiplexing

In a simple example, one or more TS are processed by a MPEG-2

multiplexor resulting in a TS Multiplex. The TS Multiplex is

forwarded over a physical bearer towards one or more Receivers

(figure 3).

+------------+                                  +------------+

|  IP        |                                  |  IP        |

|  End Host  |                                  |  End Host  |

+-----+------+                                  +------------+

|                                                ^

+------------>+---------------+                  |

+  IP           |                  |

+-------------+  Encapsulator |                  |

SI-Data |             +------+--------+                  |

+-------+-------+            |MPEG-2 TS Logical Channel  |

|  MPEG-2       |            |                           |

|  SI Tables    |            |                           |

+-------+-------+   ->+------+--------+                  |

|          -->|  MPEG-2       |                . . .

+------------>+  Multiplexor  |                  |

MPEG-2 TS             +------*--------+                  |

Logical Channel              |MPEG-2 TS Mux              |

|                           |

Other    ->+------+--------+                  |

MPEG-2  -->+  MPEG-2       |                  |

TS     --->+  Multiplexor  |                  |

---->+------+--------+                  |

|MPEG-2 TS Mux              |

|                           |

+------+--------+           +------+-----+

|Physical Layer |           |  MPEG-2    |

|Modulator      +---------->+  Receiver  |

+---------------+  MPEG-2   +------------+

TS Mux

Figure 3: An example configuration for a uni-directional service

for IP transport over MPEG-2

Expires November 2004                                      [page 11]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 Networks

July 2004

In a more complex example, the same TS may be fed to multiple MPEG-2

multiplexors and these may, in turn, feed other MPEG-2 multiplexors

(remultiplexing). Remultiplexing may occur in several places (and is

common in Scenario A of section 3.1). One example is a satellite

that provides on-board processing of the TS packets, multiplexing

the TS Logical Channels received from one or more up-link physical

bearers (TS Multiplex) to one (or more in the case of

broadcast/multicast) down-link physical bearer (TS Multiplex). As

part of the remultiplexing process, a remultiplexor may renumber the

PID values associated with one or more TS Logical Channels to

prevent clashes between input TS Logical Channels with the same PID

carried on different input multiplexes. It may also modify and/or

insert new SI data into the control plane to reflect the content

output using TS Multiplex.

In all cases, the final result is a "TS Multiplex" which is

transmitted over the physical bearer towards the Receiver.

3.4  IP Datagram Transmission

Packet data for transmission over the MPEG-2 transport multiplex is

passed to an Encapsulator, sometimes known as a Gateway.  This

receives Protocol Data Units, PDUs, such as Ethernet frames or IP

packets, and formats each into a Sub-Network Data Unit, SNDU, by

adding an encapsulation header and trailer (see section 5). The

SNDUs are subsequently fragmented into a series of TS Packets.

To receive IP packets over a MPEG-2 transport multiplex, a Receiver

needs to identify the specific TS Multiplex (physical link) and also

the TS Logical Channel (the PID value of a logical link). It is

common for a number of MPEG-2 TS Logical Channels to carry SNDUs,

and a Receiver must therefore filter (accept) IP packets sent with a

number of PID values, and must independently reassemble each SNDU.

A Receiver that simultaneously receives from several TS Logical

Channels, must filter other unwanted TS Logical Channels by

employing for example specific hardware support. Packets for one IP

flow (i.e. a specific combination of IP source and destination

addresses) must be sent using the same PID. It should not be assumed

that all IP packets are carried on a single PID, as in some cable

modem implementations, and multiple PIDs must be allowed in the

architecture. Many current hardware filters limit the maximum number

of active PIDs (e.g. 32), although if needed, future systems may

reasonably be expected to support more.

In some cases, Receivers may need to select TS Logical Channels from

a number of simultaneously active TS Multiplexes. To do this, they

need multiple physical receive interfaces (e.g., RF front-ends and

demodulators). Some applications also envisage the concurrent

Expires November 2004                                     [page 12]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

reception of IP Packets over other media that may not necessarily

use MPEG-2 transmission.

Bi-directional (duplex) transmission can be provided using a MPEG-2

transmission network by using one of a number of alternate return

channel schemes [DVB-RC]. Duplex IP paths may also be supported

using non-MPEG-2 return links (e.g. in Scenarios B-D of section

3.1). One example of such an application is that of Uni-Directional

Link Routing, UDLR [RFC3077].

The DVB family of standards currently defines a mechanism for

transporting an IP packet, or Ethernet frame using the Multi-

Protocol Encapsulation (MPE) [ETSI-DAT]. This scheme is also

supported in ATSC [ATSC-DAT; ATSC-DATG]. It allows transmission of

IP packets or Ethernet style frames in the control plane associated

with audio/video transport. Data is formatted as if it were a Table

Section. The MPE specification includes a set of optional header

components and requires decoding of the control headers.  This

processing is suboptimal for Internet traffic, since it incurs

significant receiver processing overhead and some extra link

overhead [CLC99].

+-----------------------------------------+

|Encap Header|       Subnetwork DU        |

+-----------------------------------------+

/         /                          \      \

/         /                            \      \

/         /                              \      \

+------+----------+  +------+----------+   +------+----------+

|MPEG-2| MPEG-2   |..|MPEG-2| MPEG-2   |...|MPEG-2| MPEG-2   |

|Header| Payload  |  |Header| Payload  |   |Header| Payload  |

+------+----------+  +------+----------+   +------+----------+

Figure 4: Encapsulation of an SNDU (e.g., IP packet) into a

Series of MPEG-2 TS Packets (each TS Packet carries a header

with a common Packet ID, PID, value denoting the MPEG-2 TS

Logical Channel).

3.5 Motivation

The network layer protocols to be supported by this architecture

include:

(i) IPv4 Unicast packets, destined for a single end host

(ii) IPv4 Broadcast packets, sent to all end systems in an IP

network

(iii) IPv4 Multicast packets

(iv) IPv6 Unicast packets, destined for a single end host

(v) IPv6 Multicast packets

(vi) Packets with compressed IPv4 / IPv6 packet headers (e.g.

[RFC1114; RFC2507; RFC3095])

(vii) Bridged Ethernet frames

(viii) Other (MPLS, IPv6 anycast, potential new protocols__

Expires November 2004                                      [page 13]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

The architecture will provide:

(i)  Guidance on which MPEG-2 features are pre-requisites for the IP

service, and identification of any optional fields that impact

performance/correct operation.

(ii) Standards to provide an efficient and flexible encapsulation

scheme that may be easily implemented in an Encapsulator or

Receiver. The payload encapsulation requires a type field for

the SNDU to indicate the type of packet and a mechanism to

signal which encapsulation is used on a certain PID.

(iii) Standards to associate a particular IP address with a Network

Point of Attachment (NPA) that could or could not be a MAC

Address. This process resembles the IPv4 Address Resolution

Protocol, ARP, or IPv6 Neighbour Discovery, ND, protocol [AR-

DRAFT]. In addition, the standard will be compatible with IPv6

autoconfiguration.

(iv) Standards to associate a MPEG-2 TS interface with one or more

specific TS Logical Channels (PID, TS Multiplex). Bindings are

required for both unicast transmission, and multicast reception.

In the case of IPv4 this must also support network broadcast. To

make the schemes robust to loss and state changes within the

MPEG-2 transmission network, a soft-state approach may prove

desirable.

(v)  Standards to associate the capabilities of a MPEG-2 TS Logical

Channel with IP flows. This includes mapping of QoS functions,

such as IP QoS/DSCP and RSVP, to underlying MPEG-2 TS QoS,

multi-homing and mobility. This capability could be associated

by the AR standard proposed above.

(vi) Guidance on Security for IP transmission over MPEG-2. The

framework must permit use of IPSEC and clearly identify any

security issues concerning the specified protocols. The security

issues need to consider two cases: unidirectional transfer (in

which communication is only from the sending IP end host to the

receiving IP end host) and bi-directional transfer.

Consideration should also be given to security of the TS

Multiplex: the need for closed user groups and the use of MPEG-2

TS encryption.

(vii) Management of the IP transmission, including standardised SNMP

MIBs and error reporting procedures. The need for and scope of

this is to be determined.

The specified architecture and techniques should be suited to a range

of systems employing the MPEG-2 TS, and may also suit other (sub)

networks offering similar transfer capabilities.

Expires November 2004                                      [page 14]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

The following sections 4 and 5 describe encapsulation issues.

Sections 6 and 7 describe address resolution issues for unicast and

multicast. The appendix provides some use cases.

4. Encapsulation Protocol Requirements

This section identifies requirements and provides examples of

mechanisms that may be used to perform the encapsulation of IPv4 and

IPv6 multicast and unicast packets over MPEG-2 transmission

networks.

The encapsulation protocol transports a SNDU over the MPEG-2 TS

service and provides the appropriate mechanisms to deliver this to

the Receiver IP interface. A convergence protocol typically adds

header fields before a SNDU that carry protocol control information

such as length of SNDU, Receiver address, multiplexing information,

payload type, sequence numbers etc.  The SNDU payload is typically

followed by a trailer, which carries an Integrity Check (e.g.,

Cyclic Redundancy Check, CRC). Some protocols also add additional

control information and/or padding to or after the trailer.

+--------+-------------------------+-----------------+

| Header |            SNDU         | Integrity Check |

+--------+-------------------------+-----------------+

Figure 5: Encapsulation of a subnetwork PDU (e.g. IPv4 or IPv6

packet) to form an MPEG-2 Payload Unit.

Examples of existing convergence protocols include ATM AAL5 [ITU-

AAL5], and MPEG-2 MPE [ETSI-DAT].

The existing proposals and standards for use with MPEG-2 carry

heritage from legacy implementations that reflect the limitations of

technology at their deployment.  For example a majority are more

dominated by audio/video considerations than IP requirements. This

justifies the development of a new encapsulation that will be truly

IP-centric. Carrying IP packets over a TS Logical Channel involves

several convergence protocol functions. This section briefly

describes these functions and highlights the requirements for a new

encapsulation.

4.1 Payload_Unit Delimitation

MPEG-2 indicates the start of a Payload Unit in a new TS Packet with

a "start_of_payload_unit_indicator" (PUSI) [ISO-MPEG] carried in the

4B TS Packet header. The PUSI is a 1 bit flag that has normative

meaning [ISO_MPEG] for TS Packets that carry PES Packets or PSI

data.

When the payload of a TS Packet contains PES data, a PUSI value of

'1' indicates the TS Packet payload starts with the first byte of a

Expires November 2004                                      [page 15]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

PES Packet. A value of '0' indicates that no PES Packet starts in

this TS Packet. If the PUSI is set to '1', then one, and only one,

PES Packet starts in the TS Packet.

When the payload of the TS Packet contains PSI data, a PUSI value of

'1' indicates the first byte of the TS Packet payload carries a

payload pointer that indicates the position of the first byte of the

Payload Unit (Section) being carried; if the TS Packet does not

carry the first byte of a Section, the PUSI is set to '0',

indicating that there is no payload pointer.

Using this PUSI bit, the start of the first Payload Unit in a TS

Packet is exactly known by the Receiver, unless that TS Packet has

been corrupted or lost in the transmission. In which case, the

payload is discarded until the next TS Packet is received with a

PUSI value of '1'.

The encapsulation should allow packing of more than one SNDU into

the same TS Packet and should not limit the number of SNDUs that can

be sent in a TS Packet.  In addition, it should allow an IP

Encapsulator to insert padding when there is an incomplete TS Packet

payload. A mechanism needs to be identified to differentiate this

padding from the case where another encapsulated SNDU follows.

A combination of the PUSI and a Length Indicator (see below) allows

an efficient MPEG-2 convergence protocol to receive accurate

delineation of packed SNDUs.  The MPEG-2 standard [ISO_MPEG] however

does not specify how private data should use the PUSI bit.

4.2 Length Indicator

Most services using MPEG-2 include a length field in the payload

header to allow the Receiver to identify the end of a payload unit

(e.g. PES Packet, Section, or an SNDU).

When parts of more than two Payload Units are carried in the same TS

Packet, only the start of the first is indicated by the Payload

Pointer. Placement of a Length Indicator in the encapsulation header

allows a Receiver to determine the number of bytes until the start

of the next encapsulated SNDU. This placement also provides the

opportunity for the Receiver to pre-allocate an appropriate-sized

memory buffer to receive the reassembled SNDU.

A Length Indicator is required, and should be carried in the

encapsulation header.  This should support SNDUs of at least the MTU

size offered by Ethernet (1500 bytes). Although the IPv4 and IPv6

packet format permits an IP packet of size up to 64 KB, such packets

are seldom seen on the current Internet. Since high speed links are

often limited by the packet forwarding rate of routers, there has

been a tendency for Internet core routers to support MTU values

larger than 1500 bytes. A value of 16 KB is not uncommon in the core

Expires November 2004                                      [page 16]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

of the current Internet.  This would seem a suitable maximum size

for an MPEG-2 transmission network.

4.3 Next Level Protocol Type

A key feature of the new encapsulation is to identify the type of

payload being transported (e.g., to differentiate IPv4, IPv6 etc.).

Most protocols use a type field to identify a specific process at

the next higher layer that is the originator or the recipient of the

payload (SNDU). This method is used by IPv4, IPv6, and also by the

original Ethernet protocol (DIX). OSI uses the concept of a

'Selector' for this, (e.g. in the IEEE 802/ISO 8802 standards for

CSMA/CD [LLC], although in this case a SNAP (subnetwork access

protocol) header is also required for IP packets.

A Next Level Protocol Type field is also required if compression

(e.g. Robust Header Compression, ROHC) is supported. No compression

method has currently been defined that is directly applicable to

this architecture, however the ROHC framework defines a number of

header compression techniques that may yield considerable

improvement in throughput on links which have a limited capacity.

Since many MPEG-2 transmission networks are wireless the ROHC

framework will be directly applicable for many applications. Use of

ROHC implies the need to transfer smaller SNDUs and the need for

additional processing at the Receiver to expand the received

compressed packets. The selected type field should contain

sufficient code points to support this technique.

It is thus a requirement to include a Next Level Protocol Type field

in the encapsulation header.  Such a field should specify values for

at least IPv4, IPv6, and must allow for other values (e.g., MAC-

level bridging).

4.4 L2 Subnet Addressing

In MPEG-2, the PID carried in the TS Packet header is used to

identify individual services with the help of SI tables. This was

primarily intended as a unidirectional (simplex) broadcast system.

A TS Packet stream carries either tables or one PES Packet stream

(i.e., compressed video or audio). Individual Receivers are not

addressable at this level.

IPv4 and IPv6 allocate addresses to end hosts and intermediate

systems (routers). Each system (or interface) is identified by a

globally assigned address.  ISO uses the concept of a hierarchically

structured Network Service Access Point (NSAP) address to identify

an end user process in an Internet environment.

Within a local network a completely different set of addresses for

the Network Point of Attachment (NPA) is used; frequently these NPA

addresses are referred to as Medium Access Control, MAC-level

addresses. In the Internet they are also called hardware addresses.

Expires November 2004                                    [page 17]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

Whereas network layer addresses are used for routing, NPA addresses

are primarily used for Receiver identification.

Receivers may use the NPA of a received SNDU to reject unwanted

unicast packets within the (software) interface driver at the

Receiver, but must also perform forwarding checks based on the IP

address. IP multicast and broadcast may also filter based using the

NPA, but Receivers must also filter unwanted packets at the network

layer based on source and destination IP addresses.  This does not

imply that each IP address must map to a unique NPA (more than one

IP address may map to the same NPA). If a separate NPA address is

not required, the IP address is sufficient for both functions.

If it is required to address an individual Receiver in an MPEG-2

transport system, this can be achieved either at the network level

(IP address) or via a hardware-level NPA address (MAC-address).  If

both addresses used, they must be mapped either in a static or a

dynamic way (e.g., by an address resolution process). A similar

requirement may also exist to identify the PID and TS multiplex on

which services are carried.

Using an NPA address in a MPEG-2 transport system may enhance

security, in that a particular payload may be targeted for a

particular Receiver by specifying the Receiver NPA address. This is

however only a weak form of security, since the NPA filtering is

often reconfigurable (frequently performed in software), and may be

modified by a user to allow reception of specified (or all) packets,

similar to promiscuous mode operation in Ethernet. If security is

required, it should be applied at another place (e.g. link

encryption, authentication of address resolution, IPSEC, transport

level security and/or application level security).

There are also cases where the use of a NPA is required (e.g. where

a system operates as a router) and if present this should be carried

in encapsulation header where it may be used by Receivers as a pre-

filter to discard unwanted SNDUs. The addresses allocated do not

need to conform to the IEEE MAC address format. There are many cases

where a NPA is not required, and network layer filtering may be

used. A new encapsulation protocol should therefore support an

optional NPA.

4.5 Integrity Check

For the IP service, the probability of undetected packet error

should be small (or negligible) [ID-PILC-LINK]. There is therefore

a need for a CRC to verify correctness of a received IP packet.

Such checks should be sufficient to detect incorrect operation of

the encapsulator and Receiver (including reassembly errors follo-

wing loss/corruption of TS Packets), in addition to protecting from

loss and/or corruption by the transmission network (e.g.,

multiplexors and links).

Expires November 2004                                      [page 18]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

Mechanisms exist in MPEG-2 transmission networks that may assist in

detecting loss (e.g. the 4-bit continuity counter included as

standard in the MPEG-2 TS Packet header).

A convergence protocol should use an encapsulation that provides a

strong integrity check. For ease of hardware implementation, this

check should be carried in a trailer following the SNDU. A CRC-32 is

sufficient for operation with up to a 12 KB payload, and may still

provide adequate protection for larger payloads.

4.6 Identification of Scope.

The MPE section header contains information that may be used by the

Receiver to identify the scope of the (MAC) address carried as a

NPA, and prevent TS Packets intended for one scope to be received by

another. Similar functionality may be achieved by ensuring that only

IP packets that do not have overlapping scope are sent on the same

TS Logical Channel. In some cases, this may imply the use of

multiple TS Logical Channels.

4.7 Extension Headers

The evolution of the Internet service may in future require

additional functions. A flexible protocol should therefore provide a

way to introduce new features when required, without having to

provide additional out-of-band configuration.

IPv6 introduced the concept of extension headers that carry extra

information necessary/desirable for certain subnetworks. The DOCSIS

cable specification also allows a MAC header to carry extension

headers to build operator-specific services. It is thus a

requirement for the new encapsulation to allow extension headers.

4.8 Summary of Requirements for Encapsulation

The main requirements for an IP-centric encapsulation include:

- support of IPv4 and IPv6 packets

- support for Ethernet encapsulated packets

- flexibility to support other IP formats and protocols (e.g.

ROHC, MPLS)

- easy processing by hardware devices

- low overhead/managed overhead

- a fully specified algorithm that allows a sender to pack

multiple packets per SNDU and to easily locate packet

fragments

- extensibility

- compatibility with legacy deployments

- ability to allow link encryption, when required

- capability to support a full network architecture including

data, control and management planes

Expires November 2004                                     [page 19]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

5. Address Resolution Functions

Address Resolution (AR) provides a mechanism that associates L2

information with the IP address of a system. Many L2 technologies

employ unicast AR at the sender: an IP system wishing to send an IP

packet encapsulates it and places it into a L2 frame.  It then

identifies the appropriate L3 adjacency (e.g. next hop router, end-

host) and determines the appropriate L2 adjacency (e.g. MAC address

in Ethernet) to which the frame should be sent so that the packet

gets across the L2 link.

The L2 addresses discovered using AR are normally recorded in a data

structure known as the arp/neighbor cache. The results of previous

AR requests are usually cached. Further AR protocol exchanges may be

required as communication proceeds to update or re-initialise the

client cache state contents (i.e. purge/refresh the contents [ND]).

For stability, and to allow network topology changes and client

faults, the cache contents are normally "soft state", that is, they

are aged with respect to time and old entries removed.

In some cases (e.g. ATM, FR, X.25, MPEG-2 and many more), AR

involves finding other information than the MAC address. This

includes identifying other parameters required for L2 transmission,

such as channel IDs (VCs in X.25, VCIs in ATM, or PIDs in MPEG-2

TS).

Address resolution has different purposes for unicast and multicast.

Multicast address resolution is not required for many L2 networks,

but is required for many MPEG-2 transmission networks.

5.1 Address Resolution for MPEG-2

There are three elements to the L2 information required to perform

AR for a MPEG-2 TS. These are:

(i)  A Receiver ID (e.g. a 6B MAC/NPA address).

(ii) A PID or index to find a PID.

(iii) Tuner information (e.g. a Transport Stream ID) that maps to a

set of physical layer parameters.

Elements (ii) and (iii) need to be de-referenced via indexes to the

PMT when remultiplexors are used that may renumber the PID values,

(i.e. PIDs are not intended to be end-to-end identifiers). However,

although such use is common in broadcast TV networks, many private

networks do not need to employ multiplexors that renumber PIDs (see

section 3.2).

The third element (iii) allows an AR client to resolve to a

different MPEG TS Multiplex. This is used when there are several

channels that may be used for communication (i.e. multiple

outbound/inbound links). In a mesh system, this could be used to

determine connectivity.

Expires November 2004                                      [page 20]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

This AR information is used in two ways at a Receiver:

(i) AR resolves an IP unicast or IPv4 broadcast address to the (MPEG

TS Multiplex, PID, MAC/NPA address). This allows the Receiver to set

L2 filters to let traffic pass to the IP layer. This is used for

unicast, and IPv4 subnet broadcast. Usually this is configured with

the equivalent of an "ifconfig" on the interface.

(ii) AR resolves an IP multicast address to the (MPEG TS Multiplex,

PID, MAC/NPA address), and allows the Receiver to set L2 filters

enabling traffic pass to the IP layer. This operation may need to be

performed many times based on IGMP, MLD, and Multicast Routing

protocol operation. A Receiver in a MPEG-2 TS network needs to

resolve the PID value and tuning parameters associated with a TS

Logical Channel and (at least for unicast) the destination Receiver

NPA address.

A star topology MPEG-2 TS transmission network is illustrated below,

with two terminals receiving a forward broadcast channel sent by a

Hub.  (A mesh system has some additional cases.) The forward

broadcast channel consists of a "TS Multiplex" (a single physical

bearer) allowing communication with the terminals. These communicate

using a set of return channels.

Forward broadcast

MPEG-2TS          \

----------------X       /-----\

/       /       \

| Receiver|

/----------+    A    |

/            \       /

/-----\    /              \-----/

/       \  /

|   Hub   |/

|         +\                /-----\

\       /  \              /       \

\-----/    \            | Receiver|

\-----------+    B    |

\       /

\-----/

Figure 6: MPEG-2 Transmission Network with 2 Receivers

There are three possibilities for unicast AR:

(1) A system at a terminal, A, needs to resolve an address of a

system that is at the hub;

(2) A system at a terminal, A, needs to resolve an address that is

at another terminal, B;

(3) A host at the hub needs to resolve an address that is at a

terminal. The sender (encapsulation gateway), uses AR to provide the

the MPEG TS Multiplex, PID, MAC/NPA address to be used to send

unicast, IPv4 subnet broadcast and multicast packets.

Expires November 2004                                      [page 21]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

If the Hub is as an IP router, then case (1) and (2) are the same:

the host at the Receiver doesn't know the difference.  In these

cases, the address to be determined is the L2 address of the device

at the hub to which the IP packet should be forwarded, and which

then relays the IP packet back to the forward (broadcast) MPEG-2

channel after AR (case 3).

If the hub is a L2 bridge, then case 2 still has to relay the IP

packet back to the outbound MPEG-2 channel. The AR protocol needs to

resolve the specific Receiver L2 MAC address of B, but needs to send

this on a L2 channel to the Hub.  This requires terminals to be

informed of the L2 address of other terminals.

An end host connected to the hub needs to use the AR protocol to

resolve the Receiver terminal MAC/NPA address. This requires the AR

server at the Hub to be informed of the L2 addresses of other

terminals.

5.2 Scenarios for MPEG AR

An AR protocol may transmit AR information in three distinct ways:

(i) An MPEG-2 signalling table transmitted at the MPEG-2 level

(ii) An MPEG-2 signalling table transmitted at the IP level (tbd,

no implementations of this are known)

(iii) An address resolution protocol transported over IP

(as in ND)

There are three distinct cases in which AR may be used:

A. Multiple TS-Mux and the use of re-multiplexors; e.g. Digital

Terrestrial, Satellite TV broadcast multiplexes. Many such systems

employ remultiplexors that modify the PID values associated with TS

Logical Channels as they pass through the MPEG-2 transmission

network (see Scenario A of Section 3.1).

B. Tuner configuration(s) that are fixed or controlled by some other

process. In these systems, the PID value associated with a TS

Logical Channel may be known by the Sender.

C. A service run over one TS Mux (i.e., uses only one PID, for

example DOCSIS and some current DVB-RCS multicast systems). In these

systems, the PID value of a TS Logical Channel may be known by the

Sender.

5.2.1 Table-based AR over MPEG-2

In current deployments of MPEG-2 networks, information about the set

of MPEG-2 TS Logical Channels carried over a TS Multiplex is usually

distributed via tables (service information, SI) sent using channels

assigned a specific (well-known) set of PIDs. This was originally

designed for audio/video distribution to set-top-boxes. The design

Expires November 2004                                      [page 22]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

requires access to and processing of the SI table information (which

is carried in MPEG-2 section format [ISO_DSMCC]). This scheme

reflects the complexity of delivering and co-ordinating the various

TS Logical Channels associated with a multimedia TV programme.

One possible requirement to provide TS multiplex and PID information

for IP services is to integrate additional information into the

existing MPEG-2 tables, or to define additional tables specific to

the IP service. The DVB INT and the A/92 Specification from ATSC

[ATSC-A92] are examples of the realization of such a requirement.

5.2.2 Table-based AR over IP

AR information could be carried over a TS data channel, (e.g. using

an IP protocol similar to the Service Announcement Protocol, SAP).

Implementing this independently of the SI tables, would ease

implementation, by allowing it to operate on systems where IP

processing is performed in a software driver. It may also allow the

technique to be more easily adapted to other similar delivery

networks. It also is advantageous for networks which use the MPEG-2

TS but links, but do not necessarily support audio/video services

and therefore do not need to provide interoperability with TV

equipment (e.g. links used solely for connecting IP (sub)networks).

5.2.3. Query/Response AR over IP

A query/response protocol may be used at the IP level (similar to,

or based on IPv6 Neighbor Advertisements of the ND protocol). The AR

protocol may operate over an MPEG-2 TS Logical Channel using a

previously agreed PID (e.g. configured, or communicated using a SI

table). In this case, the AR could be performed by the target system

itself (as in arp and nd). This has good soft-state properties, and

is very tolerant to failures. To find an address, a system sends a

"query" to the network, and the "target" replies.

5.3 Scoping

In some case, an MPEG-2 Transmission Network may support multiple IP

networks.  If this is the case, it is important to recognise the

context (scope) within which an address is resolved, to prevent

packets from one addressed scope leaking into other scopes.

Examples of overlapping IP address assignments, include:

(i)    Private unicast addresses (e.g. in IPv4, 10/8 prefix;

172.16/12 prefix; 192.168/16 prefix) should be confined to

one addressed area.

(ii)   Some multicast addresses, (e.g., the scoped multicast

addresses sometimes used in private networks). These are only

valid within an addressed area (examples for IPv4 include;

Expires November 2004                                      [page 23]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

239/8; 224.0.0/24; 224.0.1/24). Similar cases exist for some

IPv6 multicast addresses.

(iii)  Scoped multicast addresses.  Forwarding of these addresses

is controlled by the scope associated with the address.

IP packets with these addresses must not be allowed to travel

outside their intended scope, and may cause unexpected behaviour if

allowed to do so. In addition, overlapping address assignments can

arise using level 2 NPA addresses:

(i)    The NPA address must be unique within the TS Logical Channel.

IEEE MAC addresses used in Ethernet LANs are globally unique.

If the NPA addresses are not globally unique, the same NPA

address may be re-used by Receivers in different addressed

areas.

(ii)   The NPA broadcast address (all 1s MAC address). Traffic with

this address should be confined to one addressed area.

(iii)  Other non-IP protocols may also view sets of MAC multicast

addresses as link-local, and may produce unexpected results

if distributed across several private networks!

Reception of unicast packets destined for another addressed area may

lead to an increase in the rate of received packets by systems

connected via the network. IP end hosts normally filter received

unicast IP packets based on their assigned IP address. Reception of

the additional network traffic may contribute to processing load but

should not lead to unexpected protocol behaviour. It does however

introduce a potential Denial of Service (DoS) opportunity.

When the Receiver acts as an IP router, the receipt of such an IP

packet may lead to unexpected protocol behaviour. This also provides

a security vulnerability since arbitrary packets may be passed to

the IP layer.

5.5 AR Authentication

In many AR designs authentication has been overlooked, because of

the wired nature of most existing IP networks, which makes it easy

to control hosts physically connected. With wireless connections,

this is changing: unauthorised hosts actually can claim L2

resources. The address resolution client (i.e. Receiver) may also

need to verify the integrity and authenticity of the AR information

that it receives. There are trust relationships both ways: clients

need to know they have a valid server and that the resolution is

valid.Servers have a basic authorisation issue before they allow a

L2     address to be used.

Expires November 2004                                      [page 24]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

The MPEG-2 transmission system may also require access control to

prevent unauthorised use of the satellite bearer, however, this is

an orthogonal issue to address resolution.

5.6 Requirements for Unicast AR over MPEG-2

The requirement for AR over MPEG-2 networks include:

(i)    Use of a table based approach to promote AR scaling.  This

requires definition of the frequency of update and volume of

AR traffic generated.

(ii)   Mechanisms to install AR information at the server

(unsolicited registration).

(iii)  Mechanisms to verify AR information held at the server

(solicited responses). Appropriate timer values need to be

defined.

(iv)  An ability to purge client AR information (after IP network

renumbering, etc.).

(v)   Support of IP subnetwork scoping.

(vi)  Appropriate security associations to authenticate the sender.

6. Multicast Support

This section addresses specific issues concerning IPv4 and IPv6

multicast over MPEG-2 Transmission Networks. The primary goal of

multicast support will be efficient filtering of IP-multicast

packets by the Receiver, and the mapping of IPv4 and IPv6 multicast

addresses onto the associated PID value and TS Multiplex.  The

design should permit a large number of active multicast groups, and

should minimise the processing load at the Receiver when filtering

and forwarding IP multicast packets. For example, schemes that may

be easily implemented in hardware would be beneficial, since these

may relieve drivers and operating systems from discarding unwanted

multicast traffic.

Multicast mechanisms are used at more than one protocol level. The

upstream MPEG-2 router may forward multicast traffic on the MPEG-2

TS link using a static or dynamic set of groups. When static

forwarding is used, the set of groups may also be configured or set

using SNMP, Telnet, etc. A Receiver normally uses either IGMP/MLD or

multicast routing to establish membership tables that it uses to

dynamically set local forwarding of received groups.  In a dynamic

case, this group membership information is fed-back to the Sender to

enable it to start sending new groups and (if required) to update

the tables that it produces for multicast AR.

Appropriate procedures need to identify the correct action when the

same multicast group is available on more than one TS Logical

Channel.  This could arise when different end hosts act as senders

to contribute IP packets with the same IP group destination address.

Expires November 2004                                      [page 25]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

It may also arise when a sender duplicates the same IP group over

several TS Logical Channels (or even different TS Multiplexes), and

in this case a Receiver may potentially receive more than one copy

of the same packet. At the IP level, the host/router may be unaware

of this duplication.

6.1 Multicast AR Functions

The functions required for multicast AR may be summarised as:

(i) The Sender needs to know L2 mapping of multicast group.

(ii) The Receiver needs to know L2 mapping of multicast group.

In the Internet, multicast AR is normally a mapping function rather

than a one-to-one association using a protocol. In Ethernet, the

sender maps an IP address to a L2 MAC address, and the Receiver uses

the same mapping to determine the L2 address to set a L2

hardware/software filter entry.

A typical sequence of actions for the dynamic case is:

L3) Populate the IP L3 membership tables at the Receiver.

L3) Receivers send/forward IP L3 membership tables to the Hub

L3) Dynamic/static forwarding at hub/upstream router of IP L3

groups

L2) Populate the IP AR tables at the encapsulator gateway

(i.e. Map IP L3 mcast groups to L2 (PIDs))

L2) Distribute the AR information to Receivers

L2) Set Receiver L2 multicast filters for IP groups in the

membership table.

Multicast also introduces the concept of scoped addresses, requiring

appropriate scoping to be followed. To be flexible AR must associate

a Logical Channel (PID) not only with a group, possibly a QoS class,

and other appropriate MPEG-2 TS attributes. Performing multicast AR

at the IP level can enable providers wishing to provide

independently scoped addresses would need to use multiple Multicast

AR servers, one per multicast domain.  Explicit per group AR to

individual L2 addresses is to be avoided.

Expires November 2004                                        [page 26]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

\

|

+---+----+   +--------+

|  Tuner |---+TS Tabl | . . . .

+---+----+   +--------+       .

|                         .

+--------+   +--------+       .

| deMux  |---+PID Tabl|........

+---+----+   +--------+       :

|                         :

+--------+   +--------+   +--------+

|MPE/ULE |---+AR Cache|---+  MFIB  |

+---+----+   +--------+   +--------+

|            |            |

+---+----+   +---+----+   +---+----+

|  IP    |   |  AR    |   |IGMP/MLD|

+---+----+   +---+----+   +---+----+

|            |            |

*------------+------------+

Figure 7: Receiver Processing Architecture

6.2 Requirements for Multicast over MPEG-2

The requirements for supporting multicast include, but are not

restricted to:

(i)    Encapsulating multicast packets for transmission using a

MPEG-2 TS.

(ii)   Mapping IP multicast groups to the underlying MPEG-2 TS

Logical Channel (PID) and the MPEG-2 TS Multiplex.

(iii)  Provide AR information to allow a Receiver to locate an

IP multicast flow within an MPEG-2 TS Multiplex.

(v)   Error Reporting.

7. Summary

This document describes the architecture for a set of protocols to

perform efficient and flexible support for IP network services over

networks built upon the MPEG-2 Transport Stream (TS). It also

describes existing approaches. It focuses on IP networking, the

mechanisms that are used and their applicability to supporting IP

unicast and multicast services.

The requirements for a new encapsulation of IPv4 and IPv6 packets is

described, outlining the limitations of current methods and the need

for a streamlined IP-centric approach.

The architecture also describes MPEG-2 Address Resolution (AR)

procedures to allow dynamic configuration of the sender and Receiver

using an MPEG-2 transmission link/network.  These support IPv4 and

IPv6 services in both the unicast and multicast modes. Resolution

protocols will support IP packet transmission using both the

Expires November 2004                                      [page 27]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

Multiprotocol Encapsulation (MPE), which is currently

widely deployed, and also any new optimised encapsulation.

8. Security Considerations

When the MPEG-2 network is not using a wireline network, the normal

security issues relating to the use of wireless links for transport

Internet traffic should be considered [ID-PILC-LINK].

End-to-end security (including confidentiality, authentication,

integrity and access control) is closely associated with the end-

user assets that are protected. This close association cannot be

ensured when providing security mechanisms only within a subnetwork

(e.g. an MPEG-2 transmission network).  Several security mechanisms

that can be used end-to-end have already been deployed in the

general Internet and are enjoying increasing use. Important examples

include:

* The Secure Sockets Layer (SSL) and Transport Layer Security, TLS,

which is primarily used to protect web commerce;

* Pretty Good Privacy (PGP) and S/MIME, primarily used to protect

and authenticate email and software distributions;

* The Secure Shell (SSH), used to secure remote access and file

transfer;

* IPsec, a general purpose encryption and authentication mechanism

above IP that can be used by any IP application.

However, subnetwork security is also important [ID-PILC-LINK] and

should be encouraged, on the principle that it is better than the

default situation, which all too often is no security at all.  Users

of especially vulnerable subnets (such as radio/broadcast networks

and /or shared media Internet access) often have control over at

most one endpoint - usually a client and therefore cannot enforce

the use of end-to-end mechanisms.

A related role for subnetwork security is to protect users against

traffic analysis, i.e., identifying the communicating parties (by IP

or MAC address) and determining their communication patterns, even

when their actual contents are protected by strong end-to-end

security mechanisms (this is important for networks such as

broadcast/radio, where eaves-dropping is easy).

Where it is possible for an attacker to inject traffic into the

subnetwork control plane, subnetwork security can additionally

protect the subnetwork assets.  This threat must specifically be

considered for the protocols used for subnetwork control functions

(e.g. address resolution, management, configuration). Possible

threats include theft of service and denial of service; shared media

subnets tend to be especially vulnerable to such attacks [ID-PILC-

LINK]. Appropriate security functions must therefore be provided for

Expires November 2004                                      [page 28]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

ipdvb control protocols [ID-PILC-LINK], particularly when the

control functions are provided above the IP-layer using IP-based

protocols. Internet level security mechanisms (e.g. IPSEC) can

mitigate such threats.

In general, End-to-End security is recommended for users of any

communication path, especially when it includes a

wireless/radio/broadcast link, where a range of security techniques

already exist. Specification of security mechanisms at the

application layer, or within the MPEG-2 transmission network are the

concerns of organisations beyond the IETF. The complexity of any such

security mechanisms should be considered carefully so that it will

not unduly impact the IP operation.

8.1 Link Encryption

Link level encryption of IP traffic is commonly used in

broadcast/radio links to supplement End-to-End security (e.g.

provided by SSL, SSH, PGP, IPSec).  The encryption and key exchange

methods vary significantly, depending on the intended application.

For example, DVB-S/DVB-RCS operated by Access Network Operators

(ANO) may wish to provide their customers (Internet Service

Providers, ISP) with security services. Common targeted security

services are: terminal authentication and data confidentiality (for

unicast and multicast) between a gateway and Receivers. A common

objective is to provide the same level of privacy as terrestrial

links. An ISP may also wish to provide end-to-end security services

to the end-users (based on the well known mechanisms such as IPSec).

Therefore it is important to understand that both security solutions

(ANO-to-ISP and ISP-to-end users) may co-exist.

MPE supports optional link encryption using a pair of bits within

the MPE protocol header to indicate the use of encryption.  To

support optional link level encryption, it is recommended that a new

encapsulation also supports optional encryption of the SNDU payload.

Furthermore, it may be desirable to encrypt/authenticate some/all of

the SNDU headers.  The method of encryption and the way in which

keys are exchanged is beyond the scope of the ULE Specification.

However, the specification must provide appropriate code points to

allow such encryption to be implemented at the link layer.

9. Acknowledgments

The authors wish to thank Isabel Amonou , Torsten Jaekel, Pierre

Loyer, Luoma Juha-Pekkaand and Rod Walsh for their detailed inputs.

We also wish to acknowledge the input provided by the members of the

IETF ipdvb WG.

Expires November 2004                                      [page 29]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

10. References

10.1 Normative References

[ISO-MPEG] ISO/IEC DIS 13818-1:2000 "Information technology -- Generic

coding  of  moving  pictures  and  associated  audio  information:

Systems", International Standards Organisation (ISO).

[ETSI-DAT] EN 301 192 Specifications for Data Broadcasting, European

Telecommunications Standards Institute (ETSI).

[RFC1122] B. Braden, ed., "Requirements for Internet Hosts  -

Communication Layers", RFC 1122.

10.2 Informative References

[ATSC] A/53C, "ATSC Digital Television Standard", Advanced Television

Systems Committee (ATSC), Doc. A/53C, 2004.

[ATSC-DAT] A/90, "ATSC Data Broadcast Standard", Advanced Television

Systems Committee (ATSC), Doc. A/090, 2000.

[ATSC-DATG] A/91, "Recommended Practice: Implementation Guidelines

for the ATSC Data Broadcast Standard", Advanced Television Systems

Committee (ATSC),Doc. A/91, 2001.

[ATSC-A92] A/92  "Delivery of IP Multicast Sessions over ATSC Data

Broadcast", Advanced Television Systems Committee (ATSC), Doc. A/92,

2002.

[ATSC-G] A/54A, "Guide to the use of the ATSC Digital Television

Standard", Advanced Television Systems Committee (ATSC), Doc. A/54A,

2003.

[ATSC-PSIP-TC] A/65B, "Program and System Information Protocol for

Terrestrial Broadcast and Cable", Advanced Television Systems

Committee (ATSC), Doc. A/65B, 2003.

[ATSC-S] A/80, "Modulation and Coding Requirements for Digital TV

(DTV) Applications  over Satellite", Advanced Television Systems

Committee (ATSC), Doc. A/80, 1999.

[CLC99] Clausen, H., Linder, H., and Collini-Nocker, B., "Internet

over Broadcast Satellites", IEEE Commun. Mag. 1999, pp.146-151.

[ETSI-DAT] EN 301 192 Specifications for Data Broadcasting, European

Telecommunications Standards Institute (ETSI).

[ETSI-DVBC] EN 300 800 Digital Video Broadcasting (DVB); DVB

interaction channel for Cable TV distribution systems (CATV),

European Telecommunications Standards Institute (ETSI).

Expires November 2004                                      [page 30]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

[ETSI-DVBS] EN 301 421 Digital Video Broadcasting (DVB); Modulation

and  Coding  for  DBS  satellite  systems  at  11/12  GHz,  European

Telecommunications Standards Institute (ETSI).

[ETSI-DVBT] EN 300 744 Digital Video Broadcasting (DVB); Framing

structure, channel coding and modulation for digital terrestrial

television (DVB-T), European Telecommunications Standards Institute

(ETSI).

[ID-IPDVB-ULE] Fairhurst, G., B. Collini-Nocker, "Simple Ultra

Lightweight Encapsulation for transmission of IP datagrams over

MPEG-2/DVB networks", Internet Draft, draft-ipdvb-ule-xx.txt, Work

in Progress, IPDVB WG.

[ID-PILC-LINK] P. Karn  (ed) Advice for Internet Subnetwork

Designers, Internet Draft draft-ietf-pilc-link-design-00.txt , Work

in Progress, IETF PILC WG, BCP (with RFC-Ed).

[ID-IPDVB-AR] Fairhurst, G., M-J. Montpetit, "Address Resolution for

IP datagrams over MPEG-2 networks", Internet Draft, draft-fair-

ipdvb-ar-00.txt, Work in Progress, IP-DVB WG.

[ID-MMUSIC-IMG] Y. Nomura, R. Walsh, J-P. Luoma, J. Ott, H.

Schulzrinne, "Protocol Requirements for Internet Media Guides",

Internet Draft, draft-ietf-mmusic-img-req-XX.txt, Work in Progress,

MMUSIC WG.

[ISO-DSMCC] ISO/IEC IS 13818-6 "Information technology -- Generic

coding of moving pictures and associated audio information -- Part

6:  Extensions  for  DSM-CC  is  a  full  software  implementation",

International Standards Organisation (ISO).

[ISO-VID] ISO/IEC DIS 13818-2:1998 "Information technology -- Generic

coding of moving pictures and associated audio information: Video",

International Standards Organisation (ISO).

[ISO-AUD] ISO/IEC 13818-3:1995 "Information technology -- Generic

coding of moving pictures and associated audio information -- Part

3: Audio", International Standards Organisation (ISO).

[Ken87] Kent C.A., and J. C. Mogul, ?Fragmentation Considered

Harmful", Proc. ACM SIGCOMM, USA, CCR Vol.17, No.5, 1988, pp.390-

401.

[RFC793] Postel, J., "Transmission Control Protocol", RFC791.

[RFC1122] B. Braden, ed., "Requirements for Internet Hosts  -

Communication Layers", RFC 1122.

Expires November 2004                                      [page 31]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

[RFC1144] Jacobson, V., "Compressing TCP/IP Headers for Low-Speed

Serial Links", RFC1144.

[RFC1191] Mogul, J., Deering, S., "Path MTU Discovery", RFC 1191.

[RFC2507] Degermark, M., Nordgren, B., and Pink, S., "IP Header

Compression", RFC2507.

[RFC3077] Duros, E., W. Dabbous, H. Izumiyama, Y. Zhang, "A Link

Layer Tunneling Mechanism for Unidirectional Links", RFC3077.

[RFC3095] Bormann, C., et al, "RObust Header Compression (ROHC):

Framework and four profiles: RTP, UDP ESP and uncompressed",

RFC3095.

http://www.atsc.org/standards/Code_Point_Registry.pdf

11. Authors? Addresses

Marie J. Montpetit

MJMontpetit.com

Email: marie@mjmontpetit.com

Godred Fairhurst

Department of Engineering

University of Aberdeen

Aberdeen, AB24 3UE

UK

Email: gorry@erg.abdn.ac.uk

Web: http://www.erg.abdn.ac.uk/users/gorry

Horst D. Clausen, Bernhard Collini-Nocker, Hilmar Linder

Debartment of Scientific Computing

University of Salzburg

Jakob Haringer Str. 2

5020 Salzburg

Austria

Email: [clausen|bnocker|hlinder]@cosy.sbg.ac.at

Web: http://www.network-research.org

Expires November 2004                                        [page 32]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

12. IPR Notices

The IETF takes no position regarding the validity or scope of any

Intellectual Property Rights or other rights that might be claimed

to pertain to the implementation or use of the technology described

in this document or the extent to which any license under such

rights might or might not be available; nor does it represent that

it has made any independent effort to identify any such rights.

Information on the procedures with respect to rights in RFC

documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any

assurances of licenses to be made available, or the result of an

attempt made to obtain a general license or permission for the use

of such proprietary rights by implementers or users of this

specification can be obtained from the IETF on-line IPR repository

at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights that may cover technology that may be required to implement

this standard.  Please address the information to the IETF at ietf-

ipr@ietf.org.

13. Copyright Statement

Copyright (C) The Internet Society (2004).  This document is

subject to the rights, licenses and restrictions contained in

BCP 78, and except as set forth therein, the authors retain all

their rights.

This document and the information contained herein are provided

on an "AS IS" basis and THE CONTRIBUTORs, THE ORGANIZATIONS THEY

REPRESENT OR ARE SPONSORED BY (IF ANY), THE INTERNET SOCIETY AND

THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES,

EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY

THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS

OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A

PARTICULAR PURPOSE.

14. IANA Considerations

A set of protocols which meet these requirements, will require the

IANA to assign various numbers.  This document by itself, however,

does not require any IANA involvement.

Expires November 2004                                      [page 33]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

Appendix A: MPEG-2 Encapsulation Mechanisms

To transmit packet data over an MPEG-2 transmission network requires

that individual SNDUs (e.g. IPv4, IPv6 packets, or bridged Ethernet

Frames) are encapsulated using a convergence protocol. The following

encapsulations are currently standardised for MPEG-2 transmission

networks:

(i)     Multi-Protocol Encapsulation (MPE).

The Multi-Protocol Encapsulation, MPE, specification of DVB

[ETSI-DAT] uses private Sections for the transport of IP

packets and uses encapsulation that is similar to the IEEE

LAN/MAN standards [LLC]. Data packets are encapsulated in

datagram sections that are compliant with the DSMCC section

format for private data. Some Receivers may exploit section-

processing hardware to perform a first-level filter of the

packets that arrive at the Receiver.

The section header also includes fields, which may be used

by a Receiver to filter datagrams assigned to the same TS

Logical Channel.  This feature allows several logical

networks to be established without assigning PID values to

each of the services. Section filtering is especially well

suited for TV broadcast environments where remultiplexing

comes into play.

This encapsulation makes use of a MAC-level network point of

attachment address. The address format conforms to the

ISO/IEEE standards for LAN/MAN LLC]. The 48-bit MAC address

field contains the MAC address of the destination; it is

distributed over six 8-bit fields, labeled MAC_address_1 to

MAC_address_6. The MAC_address_1 field contains the most

significant byte of the MAC address, while MAC_address_6

contains the least significant byte.  How many of these bytes

are significant is optional and defined by the value of the

broadcast descriptor table [SI-DAT] sent separately over

another MPEG-2 TS within the TS multiplex.

MPE is currently a widely deployed scheme. Due to investments

in existing systems, usage is likely to continue in current

and future networks.

(ii)   Data Piping.

The Data Piping profile [ETSI-DAT] is a minimum overhead,

simple and flexible profile that makes no assumptions

concerning the format of the data being sent. In this

profile, the Receiver is intended to provide PID filtering,

packet reassembly according to [DVB-SIDAT-368], error

detection and optional Conditional Access (link encryption).

The specification allows the user data stream to be

unstructured or organized into packets. The specific

Expires November 2004                                      [page 34]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

structure is transparent to the Receiver. It may conform to

any protocol, e.g., IP, Ethernet, NFS, FDDI, MPEG-2 PES, etc.

(iii)  Data Streaming.

The data broadcast specification profile [ETSI-DAT] for PES

tunnels (Data Streaming) supports unicast and multicast data

services that require a stream-oriented delivery of data

packets. This encapsulation maps an IP packet into a single

PES Packet payload.

Two different types of PES headers can are selected via the

stream_id values [ISO-MPEG]. The private_stream_2 value

permits the use of the short PES header with limited

overhead, while the private_stream_1 value makes available

the scrambling control and the timing and clock reference

features of the PES layer.

Expires November 2004                                      [page 35]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

Appendix B: Address Resolution Use Cases

Consider the case where a hub server collects a table of AR

information. The collected AR information then needs to be

redistributed to clients using the forward multicast link. The

number of Receivers may range from 1 to many thousands.

(i)  Example 1 Example protocol stack for Sender Side (2 interface)

Consider a router with two logical interface (A,B) each to an IP

network which could use either private or global IP addresses.  If

AR is performed independently for each interface, the L2 addresses

used by subnetworks A,B may also overlap or be globally unique. A

separate association protocol could provide MPEG-2 other L2

information (MPEG-2 TS Logical Channels / PID mappings, etc) to the

IP systems in each of the connected IP networks. Both AR for L2

addresses and for association with TS logical channel attributes may

be performed by IP-level protocols using link-local IP multicast. A

good solution may also permit multiple servers. A simple association

protocol may only support networks that do not perform remux

renumbering of PIDs. When this needs to be supported, a L2 table

method may be used.

|

+--------+--------+

|      Port C     |

+-------------+--------+--------+-------------+

|                    Router                   |

+-----------------+---------+-----------------+

|      Port A     |         |      Port B     |

+--------+--------+         +--------+--------+

|  IP Interface 1 |         |  IP Interface 2 |

|    (subnet)     |         |    (subnet)     |

+------+----------+         +------+----------+

| AR   |   +-------+        | AR   |   +-------+

|Server|.. |   AR  +--+     |Server|.. |   AR  +--+

+------+   | Cache |  |     +------+   | Cache |  |

|  +-+-----+  |             |  +-+-----+  |

|    | Assoc  |             |    | Assoc  |

|    +--------+             |    +--------+

+-------( )--------------+----------( )-----------+

|     Encapsulator X     |    Encapsulator Y      |

+---------+--------------+-----------+------------+

|                          |

+----->------+   +----<----+

|   |

+-------( )-( )--( )-----+

|       PID PID  PID     |

|        TS-Mux          |

+---------+--------------+

| Forward Link

| (Feed)

Expires November 2004                                      [page 36]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

(ii) Example 2

Suppose we have system B and a system A. The L2 address B is (Y:q)

(i.e. Combination of MAC address, PID, TS) and the corresponding one

for A is (X:p).

|  Forward   |

+-------+  +-------+  |   Link     |  +-------+  +-------+

--+AR     +--+Encaps +--------->--------+ A     +--+AR     +-

|server |  |       |  |            |  |       |  |Client |

+-------+  +-------+  |            |  +-------+  +-------+

+------+  |            |   +------+

|A:X:p |  |            |   |p:X:A |

+------+  |            |   +------+

|            |

+-------+  +-------+  |            |  +-------+  +-------+

-+AR     +--+ B     +---------<--------+Encaps +--+AR     +-

|Client |  | Decaps|  |            |  |       |  |server |

+-------+  +-------+  |   Return   |  +-------+  +-------+

+------+  |    Link    |   +------+

|q:Y:B |  |            |   |B:Y:q |

+------+  |            |   +------+

IP addr B                         IP addr A

For B to send to A, the encapsulation gateway at B needs to resolve

the IP address of A to the l2 address of X and identify that this

must be carried over (PID,TS) of p.

A already knows its L2 address is X, this doesn't need to be

communicated. It also knows or can determine it's layer 3 IP

address. It can not determine the  (pid,TS-ID) to use, so it must be

told p. Once it knows p it can tune and set the MPEG-2 demux filter.

The corresponding addresses for B are Y and q.

How do A and B find out X and Y?

1) X can be preassigned to A (e.g. A L2 address assigned by a

Receiver manufacturer), in which case, A has to tell the AR server

at B informing it that it will be using X (register). The AR server

at B could then inform other devices by sending an AR table that

includes an entry that says A uses X (mesh). The encapsulator needs

this information, and also other clients that need to send to A.

2) B can assign X to A. B informs A that it is using X. B could then

inform others by sending an AR table that includes an entry saying A

uses X (mesh) or that B uses Y as a proxy (star).

3) It is also possible that B does not know that A is using X. To

find this out, it may query the network/a third party AR server (Y)

looking for address A. A can then respond giving its resolved

address (X), which B then uses. The way in which Y is found is to be

determined (e.g. configured, or determined by a protocol mechanism).

Expires November 2004                                      [page 37]

INTERNET DRAFT  Architecture for IP transport over MPEG-2 networks

July 2004

(iii)        Example 3

Assume a unicast (e.g. TCP) stream originates at a host, S, and is

intended for an end host, R.

The path between S and R includes an MPEG-2 link, with an

encapsulator B and a Receiver A. The Receiver, A, acts as a router.

The Encapsulator receives the IP packets from S and encapsulate them

with an SNDU destination address field of X, this is performed by

determining that A is not local to the subnetwork, but reached via

router with the IP Address A corresponding to the L2 value of X.

The packet is fragmented into TS-Packets and a PID assigned.

+-------+ +------+

+->----|AR     | |A:X   |

|   +--+Server | +------+

|   |  +-------+                 +---->  |

|   |              |             |       |

|   |              | Forward     +---->  |

|   |   +-------+  |             |       |    +-------+

|   +->-+Encaps +------------>---+---->-------+Decaps +----+->--

|   >---+       |  | Data                |    |       |    |

|  IP   +-------+  | A sent over X       |    +-------+   ++------+

|  data  +------+  |                     |     +------+   |AR     |

|        |A:X   |  | & AR Adverts (mcast)|     |X:A   | <-+Client |

|        +------+  | A uses X            |     +------+   ++------+

|                  | B uses Y            |                 |

|  Encaps receives |                     | Cached values   |

|  AR Tables       |                     | used for rx     |

|                  |                     | AR of packets   |

|  Cached values   |                     | own unicast addr|

|  used for tx     |                     | & mcast addrs   |

|                  |                     |                 |

|                  |                     |                 |

|                  | Reverse             |                 |

|       +-------+  |                     |    +-------+    |

-+--<----+Decaps +------------<----------------+Encaps +-<--+--<

Decaps |       |  |                     |    |       |

+-------+  | AR requests         |    +-------+

|  who-is-C?          |

|                     |

| & AR registration   |     IP addr A

|  I-am A I-use X     |

For A to forward packets to B, the encapsulation gateway at A.

For A to forward packets to B, the encapsulation gateway at A needs

to resolve the IP address.of B to the L2 address X. A should already

be listening on the L2 address X, because it is either the media-

resolved of its A's IP address. (For multicast, this could be L2

multicast IP address for a multicast group for which it has enabled

a filter).

Expires November 2004                                        [page 38]